IPBASE CR RIP TUTORIAL

# Introduction

This document describes the process of running IPBase to read, assess, mask and output frequency domain RIP data. It assumes that IPBase is installed on your computer and that the sample data listed in Appendix I is available. Most of the basic information on running IPBase is already described in the CR Dipole-dipole tutorial document and so only those details specific to RIP data will be presented here.

# Defining the Survey Header

IPBASE requires a header file to accompany any IP data.  You are given the option to create a new header or to read or modify an existing header file.  For the demonstration, you should select the Read Existing Header File option and select the file kate.iph. Once the file is read you will be presented with the General survey information form displayed in Figure 1.

## General Survey Information

Figure 1 Survey Design Utility - General Survey Information screen

The main pieces of information which designate this as a frequency domain gradient array survey are the Survey Mode and Array Type items. The Instrument Type and a Spacing fields are also mandatory fields and show that the data was collected using a GDP-16 receiver using an a spacing of 200m (which is the receiver dipole length). Once you are comfortable with the information displayed, click on the Done button to return to the main survey header form and click on Done again to return to IPBase.  You will then be prompted for a database name and the location to store the Access database file.

# IMPORTING THE DATA

After the Access database file has been created, you are then moved to the Import IP Data menu item (Figure 2).  The Receiver Type, Array Type, Data Type, a Spacing and Base Frequency to Read fields indicate the type of input data IPBASE is expecting, based on information provided within the Header.  For the tutorial, IPBASE is expecting GDP-16 CR data collected using 200m RIP mode.  If any of this information other than the Receiver Type is not correct, you should quit IPBase without saving and start again.

Figure 2 IP data input screen prior to data input

Next you should select the data file to import.  For the tutorial select the dump file KATE.RAW. After the file is selected IPBase will scan the file and will report any problems found. If any problems are found, you will need to edit the file to fix the problem before the data will be stored in the database. While scanning the file IPBase determines generates a list of the frequencies used in gathering the data. If there is more than one frequency, you will need to select which frequency to store in the database before the data will be saved. The file only contains data collected at 0.125 Hz so IPBase automatically stores the data.

Once IPBase starts storing data into the database, you are prompted to enter any new transmitter and receiver locations. If the database already contains these locations, you can respond No to these prompts. For the tutorial, this is a new database and so you should respond with Yes to both prompts.

The transmitter file is assumed to have the extension txc, and you should select the file KATE.TXC supplied. This file contains the following data:

1  678832.3 9647168.6  679892.9 9646108.0

2  678832.3 9646108.0  679892.9 9647168.6

The first field on each line in the file contains a transmitter id which corresponds to the transmitter id field in kate.raw.  The remaining two fields are the easting and northing coordinates of the transmitter.

The receiver file is assumed to have the extension stn, and you should select the file KATE.STN supplied. This file contains the following data:

0     674046.5   9646867.6     1650   0

1     676988.0   9644430.5   1585.0   0

3     673730.4   9646112.5   1615.4   0

4     675073.5   9652247.4   1127.8   0

5     680114.2   9653456.1    944.9   0

15    670768.4   9647333.3   1524.0   0

21    673458.6   9650075.2   1188.7   0

23    673040.5   9643164.9   2103.1   0

24    676021.6   9642386.3   1889.8   0

26    672786.5   9646611.6   1676.4   0

31    675457.9   9649224.4   1524.0   0

32    675007.0   9645060.1     2000   0

38    675750.8   9646937.8   1828.8   0

40    676529.2   9647232.5   1981.2   0

41    676674.8   9645444.6   1706.9   0

42    675784.4   9644671.9   2042.2   0

43    673672.9   9644841.0   2164.1   0

44    672470.3   9645838.0   2194.6   0

45    672342.8   9647183.5   2011.7   0

46    672899.2   9647643.4   1798.3   0

47    675086.0   9648322.0   1905.0   0

48    674290.8   9649152.5   1585.0   0

49    674367.7   9651032.2   1280.2   0

50    676958.5   9649259.0   1889.8   0

51    678410.4   9666119.7   1676.4   0

52    678420.8   9648556.4   1981.2   0

The first field on each line in the file contains a receiver id which corresponds to the transmitter id field in kate.raw.  The next two fields are the easting and northing coordinates of the transmitter. The fourth field is the station elevation and the final field is the receiver azimuth.

Once all the transmitter and receiver data has been read, IPBase continues storing the IP data into the database and then returns you to the data import screen.

# DATA STATISTICS

After the raw file has been imported, you can select the Statistics menu item where a summary table of all the data currently contained within the database is presented (Figure 3).

Figure 3 Statistical summary screen

One interesting feature of this data is the wide variation in the statistics for the raw and decoupled phases. The extreme range is a function of the differing orientation of the transmitter and receiver dipoles and is normal for RIP data.

# DATA THRESHOLDS

By selecting Display under the Thresholds menu item, you are presented with the current set of Type A User Defined Thresholds. All type A thresholds for this database are all set to null and so only the type B thresholds are displayed in Figure 4.

Figure 4 Type B thresholds

It is clear from these results that there is significant variation in the phase data. So the individual readings will need to be examined in more detail.

Figure 5 Display results screen

# DISPLAYING THE GRADIENT ARRAY DATA

Within the Thresholds screen, you can select the Display Results button to view a list view of the data within the database. When first run the default is for the average apparent resistivity to be displayed as in Figure 5. The default variable and value can be specified from the File > Preferences > Output Variable and File > Preferences > Output Value menu items.

This view of the data is quite different from the other forms of IP data. The Variable to View and Value to View are the same as before, but now you also has to select the Transmitter and Channel to View. This requirement arises because data is collected at a number of orientations at each receiver location and is also collected for a number of transmitters.

Double clicking on the data for RxID 0 in the display and clicking right on the status flag SKIPB for the decoupled phase presents you with the repeats data screen in Figure 6.

Figure 6 Repeats data screen

This screen shows that the decoupled phase standard deviation is 5.03 which is only marginally above the threshold value of 5. By examining the decoupled phase plot, spectral plot and phase plots, the repeats 1, 3 and 4  behave somewhat differently from the others. Once these repeats are switched off, the standard deviation for the decoupled phase for the remaining readings is reduced to 3.27. This value is below the threshold and so the SKIPB flags for these readings is automatically changed to PASS.

Return to the data list screen by clicking on the Done button and double click on the data cell for station 5, or alternatively click on the navigation button labelled “V” until the data for RxID is displayed. The phase plot for this location in Figure 7 is a good example of the phase flip problems that can be encountered with frequency domain data.

Figure 7 Phase plot for RxID 5 showing phase flip problem

This phase flip in the data arises because of the way in which phase values are calculated. Since phase is an angle measured in mrad any point in the complex plane can be represented by a single amplitude value but multiple phase values each separated by 2. To overcome this non-uniqueness the phase value is usually restricted to line in the range -  to +. Such a restricted range is usually adequate for most IP surveys since phase values are usually near zero. But for RIP surveys, the orientations of the receivers means that this is no longer the case. Thus when phase values near +/- are encountered the actual recorded values can cross the cut-line and the phase values can have large apparent variations even though the points they represent in the complex plane can actually be quite close together.

Another problem which can arise is that all the raw phase values can lie in one quadrant, but the decoupled phase can cross the cut line. When this happens the phase plots look like that displayed in Figure 7. IPBase attempts to correct phase reversals for display and calculation, but does not automatically fix problems with the decoupled phase.

To overcome this problem, return to the threshold menu and click on the menu item Database > Remove Phase Flips. This procedure examines all the phase values in the database and corrects any phase flips so that all phases for the repeats at each location are consistent. This process for the current database reports that 193 values were changed. Once the raw phase values have been fixed IPBase recomputes the decoupled phase values using the new raw phase values and ensures that the decoupled values are consistent with the raw values. This process can result in phases larger than + and smaller than - ,but does yield better results. Before recomputing the decoupled phase, the should check the current setting for the method for calculating the decoupled phase under the File > Preferences > Decoupled Phase Computed Using menu item. The options are to use the standard three point decoupling and the Coggon decoupling formula. In addition the decoupling method can be applied to either the magnitude and phase data directly (the standard method) or to the actual real and imaginary components. The standard methods are set as the defaults and should be adequate for most purposes, but users may like to experiment with the different methods available.

Figure 8 Phase plot for RxID 5 with phase flips removed

Once the decoupled phase has been recalculated and the thresholds reapplied, you can then display the phase plot for RxID 5 which should now look like that displayed in Figure 8. There is still a large variation in the phase data for the repeats at this location and it is left to you to examine the raw phase, decoupled phase, spectral and phase plots to try to determine a reasonable value for the raw and decoupled phase for this location.

Once all the data for the current transmitter and channel have been examined and data overrides applied, you then needs to process the other channels and transmitters. This is achieved by returning to the data list display and successively selecting the other channels and transmitters.

# Outputting data

Once the RIP data has been assessed and bad data masked out using the data thresholds and user overrides applied, the next task is to output the data into one of the standard formats used for displaying and/or modelling. The only data output option appropriate for gradient array data currently is Zonge format output.

## ZONGE OUTPUT

The Zonge output format data for the Kate database  is:

\ CRAVG : Kate.mdb (IPBASE  V2.09) 8/10/1998 USING : Mean

$ ASPACE= 200.0m

skp     Tx       Rx    PltPt    NSp  Freq Cmp Amps Magnitude    Phase    Real       Imag      %Mag   sPhz

\

2         1        0        0   1.0 0.000 Ex  4.0   3.4065e-4    27.1  1.0000e+0  0.0000e+0    0.7    5.5

2         1        0        0   1.0 0.125 Ex  4.0   3.3347e-4    86.4  1.0000e+0  8.6615e-2    0.3    1.2

2         1        0        0   1.0 0.375 Ex  4.0   3.2287e-4   200.1  9.5247e-1  1.9314e-1    0.1    3.7

2         1        0        0   1.0 0.625 Ex  4.0   3.1620e-4   303.9  9.0818e-1  2.8479e-1    0.3    5.5

2         1        0        0   1.0 0.875 Ex  4.0   3.1062e-4   401.4  8.6064e-1  3.6533e-1    0.9   12.0

2         1        0        0   1.0 1.125 Ex  4.0   3.0757e-4   495.8  8.1430e-1  4.4046e-1    1.7   16.6

2         1        0        0   1.0 0.000 Ey  4.0   6.2922e-4  3158.2  1.0000e+0  0.0000e+0    0.3    3.3

2         1        0        0   1.0 0.125 Ey  4.0   6.2765e-4  3141.0  1.0000e+0 -5.6848e-4    0.1    0.6

2         1        0        0   1.0 0.375 Ey  4.0   6.2866e-4  3112.5  1.0012e+0 -2.9164e-2    0.2    1.9

2         1        0        0   1.0 0.625 Ey  4.0   6.3488e-4  3092.1  1.0103e+0 -5.0021e-2    0.4    1.4

2         1        0        0   1.0 0.875 Ey  4.0   6.4246e-4  3077.3  1.0215e+0 -6.5812e-2    0.4    2.8

2         1        0        0   1.0 1.125 Ey  4.0   6.4945e-4  3064.7  1.0317e+0 -7.9502e-2    0.4    3.9

2         2        0        0   1.0 0.000 Ex  6.5   1.2176e-3  3155.3  1.0000e+0  0.0000e+0    0.2    0.7

2         2        0        0   1.0 0.125 Ex  6.5   1.2132e-3  3140.7  1.0000e+0 -8.5218e-4    0.1    0.2

2         2        0        0   1.0 0.375 Ex  6.5   1.2118e-3  3114.5  9.9849e-1 -2.7034e-2    0.1    0.5

2         2        0        0   1.0 0.625 Ex  6.5   1.2201e-3  3092.6  1.0045e+0 -4.9295e-2    0.2    0.6

2         2        0        0   1.0 0.875 Ex  6.5   1.2309e-3  3075.9  1.0124e+0 -6.6577e-2    0.2    2.2

2         2        0        0   1.0 1.125 Ex  6.5   1.2444e-3  3059.9  1.0223e+0 -8.3741e-2    0.1    2.4

2         1        1        1   1.0 0.000 Ex  6.5   1.8537e-2  3162.9  1.0000e+0  0.0000e+0    0.3    0.0

2         1        1        1   1.0 0.125 Ex  6.5   1.8326e-2  3165.6  1.0000e+0  2.4039e-2    0.3    0.0

2         1        1        1   1.0 0.375 Ex  6.5   1.8018e-2  3170.2  9.8309e-1  2.8119e-2    0.3    0.1

2         1        1        1   1.0 0.625 Ex  6.5   1.7862e-2  3173.3  9.7444e-1  3.0914e-2    0.3    0.2

2         1        1        1   1.0 0.875 Ex  6.5   1.7745e-2  3176.1  9.6797e-1  3.3422e-2    0.3    0.5

2         1        1        1   1.0 1.125 Ex  6.5   1.7655e-2  3178.2  9.6303e-1  3.5257e-2    0.3    0.5

2         1        1        1   1.0 0.000 Ey  6.5   7.4187e-3  3146.6  1.0000e+0  0.0000e+0    0.3    0.3

2         1        1        1   1.0 0.125 Ey  6.5   7.3894e-3  3151.5  1.0000e+0  9.9350e-3    0.3    0.1

2         1        1        1   1.0 0.375 Ey  6.5   7.3414e-3  3160.1  9.9340e-1  1.8395e-2    0.3    0.2

2         1        1        1   1.0 0.625 Ey  6.5   7.3077e-3  3166.9  9.8868e-1  2.5073e-2    0.3    0.2

2         1        1        1   1.0 0.875 Ey  6.5   7.2776e-3  3173.0  9.8444e-1  3.0916e-2    0.3    0.3

2         1        1        1   1.0 1.125 Ey  6.5   7.2490e-3  3177.9  9.8041e-1  3.5658e-2    0.3    0.5

2         2        1        1   1.0 0.000 Ex  4.5   2.2274e-2    23.1  1.0000e+0  0.0000e+0    0.0    0.5

2         2        1        1   1.0 0.125 Ex  4.5   2.2001e-2    25.2  1.0000e+0  2.5245e-2    0.0    0.2

2         2        1        1   1.0 0.375 Ex  4.5   2.1599e-2    28.4  9.8165e-1  2.7906e-2    0.0    0.1

2         2        1        1   1.0 0.625 Ex  4.5   2.1388e-2    29.9  9.7202e-1  2.9053e-2    0.0    0.2

2         2        1        1   1.0 0.875 Ex  4.5   2.1237e-2    31.4  9.6512e-1  3.0296e-2    0.0    0.3

2         2        1        1   1.0 1.125 Ex  4.5   2.1120e-2    31.7  9.5980e-1  3.0398e-2    0.0    0.3

2         2        1        1   1.0 0.000 Ey  4.5   1.1572e-3  3250.5  1.0000e+0  0.0000e+0    0.4    7.4

2         2        1        1   1.0 0.125 Ey  4.5   1.1082e-3  3180.0  1.0000e+0  3.8414e-2    0.3    3.9

2         2        1        1   1.0 0.375 Ey  4.5   1.0642e-3  3045.1  9.5649e-1 -9.2594e-2    0.2    1.2

2         2        1        1   1.0 0.625 Ey  4.5   1.0838e-3  2937.9  9.5844e-1 -1.9795e-1    0.4    3.3

2         2        1        1   1.0 0.875 Ey  4.5   1.1323e-3  2841.8  9.7689e-1 -3.0195e-1    0.4    4.4

2         2        1        1   1.0 1.125 Ey  4.5   1.1816e-3  2769.6  9.9404e-1 -3.8779e-1    0.5    5.0

………….

Appendix I

List of Tutorial files

File	Description

KATE.RAW	GDP-16 CR RIP data 0.125Hz, 200m dipole-dipole array

KATE.IPH	IPBase header ffile

KATE.TXC	Transmitter location file

KATE.STN	Receiver location file.