IPBASE CR DIPOLE-DIPOLE TUTORIAL

# Introduction

This document describes the process of running IPBase to read, assess, mask and output dipole-dipole data. It assumes that IPBase is installed on yous computer and that the sample data listed in Appendix I is available.

# Running IPBASE

Double clicking on the IPBASE icon, you will be presented will the following screen.

Figure 1 Start up screen

The first time through with a particular job or data set, you should select New Database... under the File menu item.  The first stage in creating a new database involves providing a full description of the survey which is to be stored in the database. To do this IPBase starts the Survey Design Utility module and you are presented with the form displayed in Figure 2.

# Defining the Survey Header

Figure 2 Initial Survey Design Utility screen

IPBASE requires a header file to accompany any IP data.  You are given the option to create a new header or to read or modify an existing header file.  For the demonstration, you should select the Create New Header File option where they will be presented with General Survey Information screen prompting you for information defining the survey.  Although most prompts can be left blank, to be filled in at a later date (eg. survey cost), prompts such as survey mode, array type, dipole spacing and receiver model are mandatory and you will not be able to proceed without entering this information.

## General Survey Information

Figure 3 Survey Design Utility - General Survey Information screen

In the General Survey Information screen, you are prompted for general information regarding the survey (Figure 3).  Note that you must enter (a) Survey Mode, (b) Receiver Model, (c) a-Spacing and (d) Array Type before proceeding. All other fields are optional.

### Survey Mode

The survey modes currently supported  are Complex Resistivity (or frequency domain) and Time domain

### Receiver Model

IPBASE will accept machine dump files from IPR-12, GDP-16, GDP-32, ELREC6 and HUNTEC MK IV receivers. USER DEFINED receiver type will also accept data collected using any instrument which has been converted to a generic GEOSFOFT *.DAT format.

### Array Type

The survey array types currently supported  are dipole-dipole, pole-dipole, gradient array and RIP.

## Contractor Information

Once the General Survey Information has been entered, clicking on Next presents you with the Contractor Information screen (Figure 4).  Although you could enter the information required, it would be far easier for the contractor to supply this information.  For such purpose, a stand alone Windows executable of the Survey Design Utility (SDU.EXE) has been supplied.  Contractors should be asked to use the SDU to enter appropriate information such as that displayed in Figures 4 & 5.  The output from the SDU is an IP Header file (*.IPH) which you would then input to IPBASE (Figure 2). The file DEMO.IPH contains the survey information and can be loaded using the Read Existing Header File button displayed in Figure 2.

Figure 4 Survey Design Utility - Contractor Information screen

## User Comments

By clicking on Next, users are prompted to enter any comments pertinent to the survey, i.e. location of cultural features, problems with equipment, location of outcrop (Figure 5).  The Survey Design Utility can also import any ASCII text file with these comments.

Figure 5 Survey Design Utility - User Comments screen

## Geographic Information

By clicking on Next you are prompted for geographic information (Figure 6).  As indicated on the screen, grid information can be either be hand entered or read from a file (*.ROT).  Note that the Survey Design Utility requires one point in common with the local grid and your particular map grid.  This information is required for archiving purposes.

Figure 6 Survey Design Utility - Geographic information screen.

A grid information file (HMOON.ROT) file has been provided for use with the tutorial, (refer to Appendix I).  Once the grid information has been entered, you should click on the Store in File button to save any changes and click on Next to continue.

## User Defined Thresholds

You are now presented with a User Defined Thresholds screen (Figure 7).  You may Set all thresholds to null, Set all thresholds to default, Read threshold from file, Edit thresholds or Save thresholds to file.

Figure 7 Survey Design Utility - Initial User Defined Thresholds screen

For the tutorial, you should click on Set all thresholds to null followed by the Edit threshold button.  You are now presented with the Type A UDT screen (Figure 8).

Figure 8 Survey Design Utility - Type A User Defined Thresholds screen

Type A thresholds are those that deal with an individual reading and Type B thresholds pertain to reading repeatability.  By clicking on the Show Type B button, you are presented with the Type B UDT screen (Fig 9).

Figure 9 Survey Design Utility - Type B User Defined Thresholds screen

By clicking on Done, you are moved back to the form displayed in Figure 7.  If the thresholds have been changed from the null settings, you must Save thresholds to file before selecting Next.

## Topographic Information

You are now prompted to supply a GEOSOFT *.XYZ file containing topography data (Figure 10).  If a file is supplied, you must enter a base level datum (above sea level).  You can also enter any relevant comments.

Figure 10 Survey Design Utility.  Topography information screen

The current version of IPBASE does not use topographic information but does output this data in the archive.

By selecting Done (in Figure 10) you are brought back to the form displayed in Figure 2.  You should save any changes to the header by selecting Store Current Header File, where they will be prompted for a *.IPH (IP Header) file name.  To exit from the Survey Design Utility and you should select Done, where you prompted for a name (*.MDB) and the location to store the Access database file (Figure 11).

# CREATING A DATABASE

Figure 11 Prompt for database file name and location

After the Access database file has been created, you are then moved to the Import IP Data menu item (Figure 12).  The Receiver Type, Array Type, Data Type and a Spacing fields indicate the type of input data IPBASE is expecting, based on information provided within the Header.  For the tutorial, IPBASE is expecting GDP-32 CR data collected using 100m dipole-dipole array.  If this information is not correct, you can make the appropriate changes, as in Figure 12.  This modified information will be updated in the Header.

# IMPORTING THE DATA

Next you should select the data to import.  For the tutorial select dump file 16250N.RAW.  This data comes from Honeymoon Well, Western Australia, and was collected in 1996 by ZERO at 0.125 Hz (refer to Appendix I).  The file also contains some data collected at higher frequencies.  Once the file has been selected, IPBase scans the file to ensure that it does not contain any errors and to determine what frequencies are contained in the data file. If the data file contains no errors, you should select the Tx/Rx coordinate convention used by the operator when the data was collected.  The Zonge standard convention is to record the lowest co-ordinate for the Tx and first Rx electrodes are used. As IPBase can only handle data for one frequency and the data file contains multiple frequencies, you should also select the appropriate base frequency (0.125 Hz for the tutorial).  Once this information has been supplied the Store IP data in Database button is enabled and clicking on it stores the data into the database. Note for other data file format where no coordinate convention is required and where the data file contains only one frequency, IPBase automatically stores the data in the database if the data file contains no errors.

Figure 12 Import IP screen prior to data input

After clicking on the  Store IP data in Database, button IPBASE performs the following functions:

- When a header block is encountered in the RAW file the line number defined in the file is presented to you along with other information which might be relevant to the data line, but which is not available from the data file (such as operator name). As most instruments are limited to the number and type of characters that can be used to specify the line number, IPBASE gives you the option to modify the line number to be stored in the database.

- IPBASE will allow you to import more than one instrument dump file into the database. Data are identified by line, block number, time and date.  If any duplicates are found, you are asked to ignore or overwrite.

- Once the data is imported, IPBASE displays the various data statistics.  For example, in Figure 13 we see that 12 readings out of 337 were skipped by the operator

Figure 13 IP data input screen following data input of 16250N.RAW

For the purposes of the tutorial, user should now import the 16200N.RAW data file.  This data is also 100m dipole-dipole CR data collected by Tesla-10 in 1994 using a GDP-16 instrument at a base frequency of 0.125 Hz (refer to Appendix I).  Note that Tesla-10 use the closest Tx/Rx naming convention.  Also note that Data Type, Array Type, a-Spacing and Base Frequency cannot be edited.

# DATA STATISTICS

Figure 14 Statistical summary screen

After both data sets have been imported, you can select the Statistics menu item where a summary table of all the data currently contained within the database is presented (Figure 14).

Points to note about the Statistics screen:

- The summary information may be misleading if the database contains more that one line.  You may view the statistics for individual or multiple lines by clicking on the Select Lines button and selecting the required lines.

- When the statistics of both tutorial data sets are viewed, the Rx EPROM field indicates that various EPROMS are in the database.  The correct EPROM value is indicated when only one data set is selected.  Note: No EPROM information available with GDP-16 software older than version 0527, eg. 16200N.RAW.

- Other functions of IPBASE, (i.e. output) are not influenced if you select a subset of the lines to view.

- IPBASE will only handle data collected with one base frequency.

- To be consistent with other packages, phase information has been multiplied by -1 (except for RIP surveys).

- Had users selected Open Database or Import Archive File under the File menu item in Figure 1, they would be brought directly to Statistics display.

# DATA THRESHOLDS

By selecting Display under the Thresholds menu item, you are presented with the current set of Type A User Defined Thresholds (Figure 15).  As indicated previously, Type A UDT deal with the characteristics of individual readings, while Type B deal with the repeatability of valid data for a particular plot point.

By default the current thresholds are applied when entering the Thresholds screen. If the tutorial has been followed, Type A & B UDT should be null (i.e. blank).  If not, you should select the Set Thresholds to Null option in the Thresholds menu item.  Selecting Apply Thresholds now will pass all data except those blocks and readings skipped by the contractor (Figure 15).

Figure 15 Type A UDT screen

The current thresholds for Type B (Figure 16) are presented by selecting the Show Type B button in Figure 15.

Figure 16 Type B UDT screen

Points to note about these Thresholds screens:

- The Current Thresholds and Description are provided to indicate the current mask applied on the database.

- It is envisaged that users will have a specific set of UDT for a given area.  These can be either be hand entered or read from a file via the Thresholds menu.  Thresholds could also be entered via the Survey Design Utility (refer to Figure 7).

- Under the Thresholds menu item, you always has the opportunity to save their UDT in a new file.

- At any stage you may clear the thresholds.  In addition, IPBASE provides you with two sets of “built in” UDT.  The first of these is null (the same as Clear Thresholds) and the second is a default set.

- In the Type B UDT screen (Figure 16), you are provided with a statistical summary of valid apparent resistivity, raw phase and decoupled phase.  This information is provided to give you an idea of appropriate Type B thresholds to apply.

- The Statistics screen (Figure 14) provides you with similar information for the Type A UDT.

- Total Plot Points refers to the number of unique (x, n-spacing) locations on a pseudosection.  Total data points refer to the sum of all readings taken at the plot points.  Failed refers to the number of data points and plot points failing to pass the current Type A thresholds and Type B thresholds.  In addition, the number of data and plot points flagged by the contractor are also indicated.

- The failures are inclusive, i.e. readings flagged by the contractor are counted as failures for the Type A and B UDT.

You should now select the default thresholds from the Thresholds menu.  Note the number of failures is now blank as the current thresholds have not been applied.  Click on the Apply Thresholds button.  The resulting Type A & Type B UDT screens are presented in Figures 17 & 18.

Figure 17 Type A UDT screen following default thresholds

Note that in the threshold display the first value refers to the threshold setting and the second to the number of data points failing that setting.

Figure 18 Type B UDT screen following default thresholds

We can immediately see that most of the data (306 data points out of 557) have failed the (intra) maximum raw phase standard deviation threshold of 0.5 mrad (Figure 17).  In addition, the difference between raw and decoupled phase exceeded 5 mrad for 494 readings.  In total, 515 data points have failed the default Type A thresholds and a further 4 have failed the default Type B thresholds.  As a consequence only 12 plot points passed the default UDT.  Clearly, the default UDT are too stringent on the Honeymoon Well data set.

Participants at a Western Region IP Workshop were asked to design a set of realistic thresholds for the Honeymoon Well environment.  These can be found in the WR.UDT file.  [Under the Thresholds menu item, you should now read in the WR.UDT file, refer to Figure 19].

The maximum EM coupling threshold was relaxed as large differences between raw and decoupled phase are observed.  The strong EM coupling at Honeymoon Well is due to the 50-100m of conductive overburden, often less than 1 ohm m.  High SEM values due to telluric noise were observed during acquisition.  To “allow” for this noise, the Western Region geophysicist set the maximum raw phase standard deviation threshold to null. The other significant change to the thresholds is that the minimum number of repeats has been reduced to 2.  The contractor was not specifically asked to acquire more than two readings during the 1994 survey.

The resulting Type A & B UDT screens are presented in Figures 19 & 20.

Figure 19 Type A UDT screen following Western Region thresholds

Figure 20 Type B UDT screen following Western Region thresholds

Just over 20 plot points out of 145 are deemed to be too noisy based on the Western Region thresholds.

# DISPLAYING THE IP DATA

Within the Thresholds screen, you can select the Display Results button to view a pseudosection of the data within the database. On this screen select Raw Phase and No. Valid Readings and scroll right to display the following part of the pseudosection for line 16200N (Figure 21).

Figure 21 Display results screen

Note that in this display all seven readings taken at 16200N 10150E n=3 have failed the western Region thresholds.

You can select the following Variables to View as a pseudosection:

Raw Phase	Fundamental: Magnitude

Decoupled Phase	Harmonic 3: Magnitude	Harmonic 3: Phase

Apparent Resistivity	Harmonic 5: Magnitude	Harmonic 5: Phase

SEM (of raw phase)	Harmonic 7: Magnitude	Harmonic 7: Phase

Self Potential	Harmonic 9: Magnitude	Harmonic 9: Phase

Contact Resistance	EM Coupling	Transmitter Current

You can also select the following values for each of the variables above:

Mean			Standard deviation

Median			Standard Error Mean

Minimum			No Valid Readings

Maximum			Reciprocal Count

Maximum - Minimum (i.e. data range)			Total Readings

Absolute difference between Mean and Median

As an example, the median decoupled phase for the portion of the pseudosection shown in Figure 21 is displayed in Figure 22.  Plot points with the value TN (too noisy) indicate that the point has failed the current UDT.

Figure 22 Pseudosection of median decoupled phase

Note that 16200N 10150E n=3 is annotated TN (too noisy).

The Value to view - No. Valid Readings is provided for you to gauge where on the pseudosection data are failing the current UDT. No. Valid Readings is expressed as the number of readings failing the thresholds out of the total number of readings.  For example, all seven readings failed the default thresholds at 16200N 10150E n=3, (Figure 21).

Figure 23 Pseudosection of reciprocal count self potential

The Value to view - Reciprocal Count only shows the number of valid readings collected for each orientation of the TX and Rx  at each plot point.  As all readings fail for Decoupled Phase at 10150E n=3, the reciprocal counts would both be zero if the Variable to View remains as Decoupled Phase. However SP data does not have any thresholds to pass, so changing the Variable to View to SP will allow the actual reciprocal counts to be viewed for the entire line.  In this case, Figure 23 indicates that at 10150 n=3, four were collected with the Tx to the left of the plot point and three to the right. For further details on the values and orientation of each reading, click once on the reading value in the psuedosection to bring up the grid of readings for that location.

Within the Display Results screens (Figures 21-23) IPBASE gives you the opportunity to override any of the threshold settings at any plot point location.  By double clicking the left mouse button on the 16200N 10150E n=3 plot point, you are presented with the repeats reading screen in Figure 24.

Figure 24 Reading Repeats and User overrides screen for 10150 n=3

Seven readings were collected at this plot point.  The DATA TYPE column indicates that the contractor thought all readings were valid.  You can override the DATA TYPE setting by double clicking on any DATA TYPE cell.  Double clicking again will undo the change.  Valid DATA TYPE values are listed below with a brief description.

- RAW		Valid reading

- SKIP		Initially RAW but has been overridden by you

- FLAG		Reading skipped by contractor

- USEF	Initially FLAG but has been overridden by you

Along with the apparent resistivity, raw and decoupled phase values, there is a STATUS cell for each reading (Figure 24).  STATUS indicates the status of each reading after passing through the current UDT.  Listed below are the valid STATUS values, followed by a brief description.  As with DATA TYPE, you may override the STATUS setting by double clicking on individual cells.

- PASS	Reading has passed current UDT.  Only applicable to RAW and USEF DATA TYPES.

- USEA	Reading failed the current Type A thresholds but has been overridden by user.

- USEB	Reading failed the current Type B thresholds but has been overridden by user.

- SKIP	Reading skipped by you and therefore has been ignored.

- FLAG	Reading skipped by contractor and therefore has been ignored.

- SKPA	Reading failed the current Type A thresholds.  Only available for RAW and USEF DATA TYPES.

- SKPB	Reading failed the current Type B thresholds.  Only available for RAW and USEF DATA TYPES.

Figure 24 indicated that for plot point 10150 n=3, all seven readings have passed the Type B thresholds for apparent resistivity.  Unfortunately, there is a big discrepancy in raw phase between the first four and the last three readings and so this plot point has failed the raw and decoupled phase repeatability thresholds.  Presumably, these two groups of readings are reciprocal groups.

As indicated previously, any reading failing a Type B raw phase threshold also fails the decoupled phase threshold.  As a consequence, all seven readings have failed the Type B decoupled phase thresholds.  By scrolling to the right, you can see that the first four readings are considerably noisier than the last readings.  An asterisk is used to denote readings that fail a Type A threshold (eg. Tx current).

## Navigating the Repeats Reading Screen

The plot point and the n level for the data displayed in the repeats form is initially selected by double-clicking on the appropriate location on the displayed pseudosection. Once the data at this location has been assessed and any overrides applied another location can be viewed by clicking on the Done button to return to the pseudosection display and then double-clicking on the next location required. Alternatively the navigation buttons to the left of the cancel button can be used to select the next available plot-point with data in the neighbourhood of the current location. The layout of the buttons is similar to a compass with the buttons in the North and South positions selecting the next data up and down the column (ie at the same plot-point but changing the n-level), the buttons in the East and West position selecting the next data right or left in the row (ie at the same n-level but changing the plot-point). Similarly the buttons in the Northeast and Southwest move along the diagonal sloping up to the right and the buttons in the Northwest and Southeast move along the diagonal sloping up to the left. When any of the buttons is clicked and there is no more data in the selected direction, IPBase wraps around the next column (or row or diagonal) in the direction being searched and continues until a location with data is found.

## Display Plot

Figure 25 Plot of apparent resistivity at 10150 n=3.  All readings appear valid

Before we override  the DATA TYPE or STATUS of any reading, it would be a good idea to view all repeats graphically.  By clicking on the Display Plot button, you can graphically examine variability of apparent resistivity, raw phase, decoupled phase, spectral character and phase versus frequency.  For apparent resistivity, raw phase and decoupled phase, the plot presented has the variable on the y-axis and reading number (or repeat) on the x-axis (Figures 25-27).  All readings (passed and failed) for the plot point are shown.

Figure 26 Plot of raw phase at 10150 n=3.  The offset due to reciprocals is obvious

Figure 27 Plot of decoupled phase at 10150 n=3.  Surprisingly the decoupled phase appears repeatable

The list on the right-hand side of the screen displays the DATA TYPE for each reading, with a tick designating that the reading has passed the relevant thresholds.  You can override the DATA TYPE status by clicking on check box for the relevant reading to toggle the reading on and off. This function only operates on the DATA TYPE status column and toggles the status between RAW and SKIP (or FLAG and USEF).

When selecting Spectral Plot type, you are presented with a Real versus Imaginary plot (Figure 28).  Although the spectral traces are correct, the x and y-axes are incorrectly labelled.  The intersection of the x & y axes is at 1000, 0 on the real/imaginary plane, with the Imaginary axis pointing negative upwards.  Note that the values on the x-axis need to have 1000 added.

Figure 28 Spectral and phase plots for 10150 n=3.  The differences between the reciprocal readings are immediately obvious.  Does the decoupling algorithm always work this well?  Is it correct?

As all the decoupled phase readings appear to be valid, you should select Display Data and then “turn on” STATUS to pass all the readings, as indicated in Figure 29. This can be done by double clicking each SKPB status flag for the Decoupled Phase readings, or by clicking on the first SKIPB flag with the right mouse button.

Figure 29 User overrides for plot point 10150 n=3.  Compare to Figure 24

By clicking on Done, you are moved back to original Display Results screen (Figure 21-23).  Note the TN posting for median decoupled phase at 10150 n=3 has been replaced with 9.20.

## Histogram

If the selected Value to View is anything other than Count or Reciprocal Count, you may view a histogram of the selected variable.  The histogram of median decoupled phase is presented as Figure 30.  By clicking on the Display Data button, you are returned to the Display Results screen.

Figure 30 Histogram display of valid median decoupled phase

## Colour Section

A simple colour coded pseudosection of the valid data can be displayed by selecting the Colour Section button.  This option is only available when the selected Value to View is other than Count or Reciprocal Count (eg. Figure 22).  Data points failing thresholds are ignored. This display is most useful when used to display the data range or standard deviation data as those locations with large variations can be quickly located.

## Reapplying Thresholds

If you decide that you would like to change the current thresholds, IPBASE retains the current user overrides and applies them after applying the new thresholds.  If your overrides are no longer appropriate, you can either globally clear the overrides by using the menu item Database > Options > Clear Overrides in database, or by returning to the Display Results screen and resetting the overrides individually. To assist in locating those plotpoints at which user overrides have been applied, click on the Select Data button the pseudosection display to bring up the form shown in Figure 31.

Figure 31 Select Data Form

This form allows you to define a number of selection criteria for data points in the pseudosection. To locate points at which user overrides have been applied, turn on the Override Type A and Override Type B check boxes and ensure all other check boxes have been turned off. Clicking on the Apply button returns you to the pseudosection display with the change that any plot point with a reading satisfying any of the selection criteria will be displayed in red. This makes locating readings with user overrides much simpler.

# Outputting data

Once the data has been assessed and bad data masked out using the data thresholds and user overrides, the next task is to output the data into one of the standard formats used for displaying and/or modelling. Data passed by IPBASE can be output in the following formats;

- GEOSOFT DAT

- GEOSOFT pseudosection,

- UBC inversion (DCIP2D),

- XYZ data file,

- Zonge AVG (suitable for import into the Zonge smooth model inversion package S2DIP), and

- IPBASE archive (*.IPA).

## A. GEOSOFT

To output data from the database into a Geosoft format file click on the menu item Output > Geosoft to bring up the GEOSOFT output screen is presented in Figure 32.

Once the screen is displayed, the first task is to select the data variable and value to output fields. The following data can be output in Reduced GEOSOFT (pseudosection) format:

Raw Phase	Fundamental: Magnitude

Decoupled Phase	Harmonic 3: Magnitude

Apparent Resistivity	Harmonic 5: Magnitude

SEM (of raw phase)	Harmonic 7: Magnitude

Self Potential	Harmonic 9: Magnitude

Contact Resistance	Harmonic 3: Phase

EM Coupling	Harmonic 5: Phase

Transmitter Current	Harmonic 7: Phase

Harmonic 9: Phase

In addition the following values can be output:

Mean		Standard deviation

Median		Standard Error Mean

Minimum		No Valid Readings

Maximum		Reciprocal Count

Maximum - Minimum (i.e. data range)		Total Readings

Absolute difference between Mean - Median

Figure 32 GEOSOFT DAT output screen

To select a data variable to output click in the first available blank cell in the column entitled Output Variable. This brings up a list box from which the required variable can be selected. Clicking on the variable name in this list box enters the variable into the list using Mean as the output value. To change the output value, click on the cell containing the value to be changed to bring up a list box of available output values and select the required method from the list. If the wrong variable is selected, simply click on it again and select the correct name from the list box. Clicking on the Clear Last Variable button clears the last defined variable from the output list, and clicking on the Clear All Variables clears the entire list. To make output of standard datasets faster, an output list can be defined and written to a file using the Write List to File button. Once a list has been saved it can be recalled using the Read List From File button.

Once a list of variables has been defined, the next step is to select the particular Geosoft format to create. The available options are:

DAT (repeats reduced)

DAT (repeats not merged)

Psuedosection (merged)

For both the DAT (repeats reduced) and Psuedosection (merged) output formats, a single data value is output for each plot-point/n level location for which there is one or more valid data reading. If there are multiple readings, the output method specified for that variable is applied and the resulting value output to the file. If there are readings at a plot-point/n level location but none of them are valid an asterisk is output to the data file. For the DAT (repeats not merged) format each valid repeat is output to the file and the specified output value is ignored.

The file produced using the DAT (repeats reduced) format with the specifications displayed in Figure 32  is:

IP DATA FROM : monster.mdb (IPBASE  V2.09) 7/10/1998 MASK:wr.udt

LINE:16200N ARRAY:DPDP DIPOLE:100 UNITS:M

F1        F2        M1        M2       RES       0IP

9900.0   10000.0    9500.0    9600.0      2.80      3.15

9900.0   10000.0    9600.0    9700.0      2.41      0.30

10000.0   10100.0    9500.0    9600.0      4.44      4.70

9900.0   10000.0    9700.0    9800.0      1.95      0.50

10000.0   10100.0    9600.0    9700.0      3.60      2.05

10100.0   10200.0    9500.0    9600.0      2.88      4.10

10000.0   10100.0    9700.0    9800.0      2.38      1.50

10100.0   10200.0    9600.0    9700.0      2.43      1.47

10200.0   10300.0    9500.0    9600.0      3.30      6.50

10000.0   10100.0    9800.0    9900.0      1.93      1.55

10100.0   10200.0    9700.0    9800.0      1.72      0.87

10200.0   10300.0    9600.0    9700.0      2.82      3.80

10100.0   10200.0    9800.0    9900.0      1.42      0.73

10200.0   10300.0    9700.0    9800.0      2.07      3.23

10300.0   10400.0    9600.0    9700.0      5.64     12.63

10100.0   10200.0    9900.0   10000.0      1.43      0.17

10200.0   10300.0    9800.0    9900.0      1.79      2.33

10300.0   10400.0    9700.0    9800.0      4.31     12.10

10200.0   10300.0    9900.0   10000.0      1.88      1.03

10300.0   10400.0    9800.0    9900.0      3.88     10.80

10400.0   10500.0    9700.0    9800.0      4.71     12.65

10200.0   10300.0   10000.0   10100.0      1.56      0.17

9900.0   10000.0   10300.0   10400.0      4.25      9.36

10400.0   10500.0    9800.0    9900.0      4.36     12.30

10000.0   10100.0   10300.0   10400.0      4.00         *

9900.0   10000.0   10400.0   10500.0      5.06         *

10500.0   10600.0    9800.0    9900.0      3.91     10.30

10100.0   10200.0   10300.0   10400.0      2.09         *

10000.0   10100.0   10400.0   10500.0      5.20         *

9900.0   10000.0   10500.0   10600.0      4.65         *

10100.0   10200.0   10400.0   10500.0      2.54         *

10000.0   10100.0   10500.0   10600.0      4.94         *

9900.0   10000.0   10600.0   10700.0      3.62         *

10200.0   10300.0   10400.0   10500.0      1.89         *

10100.0   10200.0   10500.0   10600.0      2.33         *

10000.0   10100.0   10600.0   10700.0      4.02         *

10200.0   10300.0   10500.0   10600.0      1.83         *

10100.0   10200.0   10600.0   10700.0      1.94         *

10000.0   10100.0   10700.0   10800.0      5.78      7.40

10300.0   10400.0   10500.0   10600.0      1.84      5.30

10200.0   10300.0   10600.0   10700.0      1.61     11.03

10100.0   10200.0   10700.0   10800.0      2.74      7.80

10300.0   10400.0   10600.0   10700.0      1.94      5.68

10200.0   10300.0   10700.0   10800.0      2.39     11.20

10100.0   10200.0   10800.0   10900.0      2.71      7.10

10400.0   10500.0   10600.0   10700.0      1.30         *

10300.0   10400.0   10700.0   10800.0      3.18      6.57

10200.0   10300.0   10800.0   10900.0      2.49     10.23

10400.0   10500.0   10700.0   10800.0      2.43     -2.40

10300.0   10400.0   10800.0   10900.0      3.59      6.07

10200.0   10300.0   10900.0   11000.0      2.80     10.00

10500.0   10600.0   10700.0   10800.0      1.31     -0.37

10400.0   10500.0   10800.0   10900.0      3.08     -3.63

10300.0   10400.0   10900.0   11000.0      4.20      6.13

10500.0   10600.0   10800.0   10900.0      2.11     -1.60

10400.0   10500.0   10900.0   11000.0      3.72     -4.15

10300.0   10400.0   11000.0   11100.0      2.65      7.55

10600.0   10700.0   10800.0   10900.0      1.37     -0.17

10500.0   10600.0   10900.0   11000.0      2.64     -2.63

10400.0   10500.0   11000.0   11100.0      2.33     -3.17

10600.0   10700.0   10900.0   11000.0      1.70     -1.43

10500.0   10600.0   11000.0   11100.0      1.71     -2.47

10600.0   10700.0   11000.0   11100.0      1.18     -1.70

Note that the resistivity column is named RES and the decoupled phase is named 0IP. The full list of output column names is:

Apparent Resistivity	RES

Decoupled Phase	0IP

SEM (of raw phase)	SEM

Self Potential	SP

EM Coupling	EMC

Transmitter Current	TXI

Contact Resistance	CRX

Raw Phase	1IP

Harmonic 3: Phase	3IP

Harmonic 5: Phase	5IP

Harmonic 7: Phase	7IP

Harmonic 9: Phase	9IP

Fundamental: Magnitude	MAG

Harmonic 3: Magnitude	3MG

Harmonic 5: Magnitude	5MG

Harmonic 7: Magnitude	7MG

Harmonic 9: Magnitude	9MG

The resistivity file produced using the Psuedosection (merged) format with the specifications displayed in Figure 32  is:

IP DATA FROM : HMoon.mdb (IPBASE  V2.09) 7/10/1998 DATA : Mean Apparent Resistivity (ohm.m) UNITS : ohm-mLINE:16200N ARRAY:DPDP DIPOLE:100 UNITS:M MASK:wr.udt

stn      filt       n=1       n=2       n=3       n=4       n=5       n=6       n=7       n=8

10850.0         *         *         *      1.18         *         *         *         *         *

10800.0         *         *      1.70         *      1.71         *         *         *         *

10750.0         *      1.37         *      2.64         *      2.33         *         *         *

10700.0         *         *      2.11         *      3.72         *      2.65         *         *

10650.0         *      1.31         *      3.08         *      4.20         *         *         *

10600.0         *         *      2.43         *      3.59         *      2.80         *         *

10550.0         *      1.30         *      3.18         *      2.49         *         *         *

10500.0         *         *      1.94         *      2.39         *      2.71         *         *

10450.0         *      1.84         *      1.61         *      2.74         *         *         *

10400.0         *         *      1.83         *      1.94         *      5.78         *         *

10350.0         *      1.89         *      2.33         *      4.02         *         *         *

10300.0         *         *      2.54         *      4.94         *      3.62         *         *

10250.0         *      2.09         *      5.20         *      4.65         *         *         *

10200.0         *         *      4.00         *      5.06         *      3.91         *         *

10150.0         *      1.56         *      4.25         *      4.36         *         *         *

10100.0         *         *      1.88         *      3.88         *      4.71         *         *

10050.0         *      1.43         *      1.79         *      4.31         *         *         *

10000.0         *         *      1.42         *      2.07         *      5.64         *         *

9950.0         *      1.93         *      1.72         *      2.82         *         *         *

9900.0         *         *      2.38         *      2.43         *      3.30         *         *

9850.0         *      1.95         *      3.60         *      2.88         *         *         *

9800.0         *         *      2.41         *      4.44         *         *         *         *

9750.0         *         *         *      2.80         *         *         *         *         *

Note that when multiple data variables have been selected for Psuedosection  format output, each variable is output to a separate file using the line number as the root file name and the data type as the file extenstion. So the resistivity file displayed above is named 16200n.res and the decoupled phase is output to 16200n.0ip.

The final option available when generating Geosoft output files is the Available Lines list box. If data for only one line is to generated, simply select the required line from the list box and click on the Output to File button. If more than one line is stored in the database then all lines can be written to file by selecting ALL from the line list and then clicking on the Output to File button. For DAT format output all the available lines are written to a single file with each line delimited in the file with it’s own LINE line. For pseudosection output data is for each line written to a separate file using the line number as the file root name.

## B. ZONGE

The Zonge output screen is displayed in Figure 33 and is accessed by clicking on the menu item Output > Zonge.  The output files created are a Zonge average file(*.AVG) and a stripped down model file (*.MDE).  These files are required by the Zonge smooth model inversion input program, TS2DIP.EXE.

Figure 33 Zonge output screen

There are fewer options available for generating Zonge data files. To generate an output file select the line to output, value to output and file location and then click on the Output to File button. The main option available is the method used to generate the decoupled phase in the output file. Two methods are available:

Decouple using phase/amplitude

Decouple using real/imaginary components

By default the Decouple using real/imaginary components is not checked so the decoupled phase is calculated using the standard phase/amplitude components. In some extreme cases GDP files can have pi flips in the phase data which results in the decoupled phase being wildly incorrect. In such cases converting the phase and amplitude to real/imaginary values and then decoupling these values produces better results. (Note such phase flips are automatically corrected by IPBase so the standard decoupling method will produce correct decoupled values, however the real/imaginary decoupling method may be useful for decoupling data with particular forms of noise).

The file produced using the DAT (repeats reduced) format with the specifications displayed in Figure 32  is:

\ IPBASE  V2.09 : monster.mdb Created on : 7/10/1998 USING : Mean Survey Type : Dipole-Dipole

$ ASPACE= 100.0m

skp     Tx       Rx    PltPt    NSp  Freq Cmp Amps Resistivity  Phase    Real       Imag      %Rho   sPhz

\

2   9500.00  9900.00  9750.00   3.0 0.000 Ex 20.0   2.7974e+0     3.2  1.0000e+0  0.0000e+0    0.1    0.1

2   9500.00  9900.00  9750.00   3.0 0.125 Ex 20.0   3.7792e+0    17.8  1.0000e+0  1.7752e-2    0.1    0.0

2   9500.00  9900.00  9750.00   3.0 0.375 Ex 20.0   3.7667e+0    46.8  9.9577e-1  4.6686e-2    0.1    0.1

2   9500.00  9900.00  9750.00   3.0 0.625 Ex 20.0   3.7560e+0    75.6  9.9118e-1  7.5076e-2    0.1    0.0

2   9500.00  9900.00  9750.00   3.0 0.875 Ex 20.0   3.7439e+0   104.1  9.8546e-1  1.0291e-1    0.1    0.0

2   9500.00  9900.00  9750.00   3.0 1.125 Ex 20.0   3.7318e+0   132.1  9.7900e-1  1.3008e-1    0.1    0.1

2   9500.00 10000.00  9800.00   4.0 0.000 Ex 23.0   4.4440e+0     4.7  1.0000e+0  0.0000e+0    0.1    0.1

2   9500.00 10000.00  9800.00   4.0 0.125 Ex 23.0   3.4521e+0    23.5  1.0000e+0  2.3504e-2    0.1    0.0

2   9500.00 10000.00  9800.00   4.0 0.375 Ex 23.0   3.4339e+0    60.4  9.9319e-1  6.0062e-2    0.1    0.0

2   9500.00 10000.00  9800.00   4.0 0.625 Ex 23.0   3.4184e+0    96.4  9.8592e-1  9.5288e-2    0.1    0.0

2   9500.00 10000.00  9800.00   4.0 0.875 Ex 23.0   3.4026e+0   131.8  9.7737e-1  1.2952e-1    0.1    0.1

...

## C. UBC OUTPUT

Figure 34 UBC inversion output screen

The UBC output screen is similar to the GEOSOFT and Zonge screens (Figure 33).  The major differences are:

- By default apparent resistivity and phase data are output.  You are given the choice of raw or decoupled phase.

- You must define the data uncertainty either as the standard deviation of the valid readings plus a constant or as a percentage of the reading plus a constant.

- You must supply a root file name no longer than six characters.

- The output files for the example shown in Figure 34 will be 16200PT.OBS (resistivity data expressed as potentials .e. volts) and 16200IP.OBS

The potential file produced using the specifications displayed in Figure 34  is:

IP DATA FROM : monster.mdb (IPBASE  V2.09) 7/10/1998 DATA : Mean Apparent Resistivity

12    1    1

9500    9600       4

10000.0  9900.0  1.484088e-04  7.420442e-06

10100.0 10000.0  1.178814e-04  5.894072e-06

10200.0 10100.0  4.370610e-05  2.185305e-06

10300.0 10200.0  3.130990e-05  1.565495e-06

9600    9700       5

10000.0  9900.0  3.201008e-04  1.600504e-05

10100.0 10000.0  1.909832e-04  9.549161e-06

10200.0 10100.0  6.438433e-05  3.219216e-06

10300.0 10200.0  4.274351e-05  2.137176e-06

10400.0 10300.0  5.347200e-05  2.673600e-06

9700    9800       6

10000.0  9900.0  1.035606e-03  5.178032e-05

10100.0 10000.0  3.155781e-04  1.577890e-05

10200.0 10100.0  9.146747e-05  4.573374e-06

10300.0 10200.0  5.495526e-05  2.747763e-06

10400.0 10300.0  6.526973e-05  3.263487e-06

10500.0 10400.0  4.460508e-05  2.230254e-06

9800    9900       6

10100.0 10000.0  1.021701e-03  5.108503e-05

10200.0 10100.0  1.886683e-04  9.433417e-06

10300.0 10200.0  9.470832e-05  4.735416e-06

10400.0 10300.0  1.030317e-04  5.151584e-06

10500.0 10400.0  6.602658e-05  3.301329e-06

10600.0 10500.0  3.708619e-05  1.854310e-06

9900   10000       6

10200.0 10100.0  7.606581e-04  3.803290e-05

10300.0 10200.0  2.498792e-04  1.249396e-05

10400.0 10300.0  2.255063e-04  1.127532e-05

10500.0 10400.0  1.342610e-04  6.713053e-06

10600.0 10500.0  7.048798e-05  3.524399e-06

10700.0 10600.0  3.433192e-05  1.716596e-06

10000   10100       6

10300.0 10200.0  8.250251e-04  4.125125e-05

10400.0 10300.0  5.305369e-04  2.652685e-05

10500.0 10400.0  2.761219e-04  1.380610e-05

10600.0 10500.0  1.309387e-04  6.546934e-06

10700.0 10600.0  6.087487e-05  3.043744e-06

10800.0 10700.0  5.471239e-05  2.735619e-06

## D: XYZ OUTPUT

The XYZ output option is not relevant for Dipole-dipole data. It is intended for use with gradient array and RIP surveys to generate located data which is suitable for generating images of the survey data.

## E. ARCHIVE OUTPUT

The archive output screen (Figure 35) shows you the information stored in the current header file (*.IPH).  This display is provided so you can check that all the required information is present and valid.

Figure 35 Archive output screen.  The output archive file is and ASCII format

It is important for later retrieval that the header information accompanies the archived data. Therefore the first portion of the archive output file (*.IPA) is the header file.  The second part of the *.IPA file is all the data in comma delimited format.  The data is sorted by line number, then plot point and then n-spacing order and is accompanied by statistical information of the valid data.  If a particular plot point has failed the current UDT, then the archive statistics will show a #N/A.

IPBASE is able to import the IPA file.  This option can be found under the File menu item.

The archive output for the above database is :

!--------------------------!

! Start of Survey Header   !

!--------------------------!

Header file for HMoon.mdb (IPBASE  V2.09) 7/10/1998

!----------------------------!

! General Survey Information !

!----------------------------!

Data type                  : Complex Resistivity

Array type                 : Dipole-Dipole

A Spacing                  : 100

Survey Name                : Monster Gold Mine

Survey Contact             :

Survey Report Number       :

Cost Code                  : G1234

Survey Line Number         : 16200N,16250N

Survey Start Date          : 05/08/1994

Survey Finish Date         : 15/05/1996

Survey Cost                : 0

Survey Production Cost     : 0

Survey Production Rate     : 0

!----------------------------!

! Contractor Information     !

!----------------------------!

Survey Contractor          : Zonge Engineering and Research Organization

Survey Operator Name       : Justin Freytag

Survey Job Number          : 307

Number of Production Days  : 0

Number of Production Hours : 0

Survey Generator Model     : GMG-30

Survey Tx Model            : GGT-30

Survey Tx Serial           : 2001

Survey Rx Model            : GDP-16 , GDP-32

Survey Rx Serial           : 3027

Rx EPROM                   : ZMT-32 Z203b

Base Frequency             : .125

!----------------------------!

! Survey Line Information    !

!----------------------------!

Line Number                : 16200N

Line Array Type            : Dipole-Dipole

Line Data Type             : Complex Resistivity

Line A Spacing             : 100

Line Base Frequency        : .125

Line Job Number            : 307

Line Operator              : Justin Freytag

Line Tx Model              : GGT-30

Line Tx Serial             : 2001

Line Rx Model              : GDP-16

Line Rx Serial             : 3027

Line Rx EPROM              :

Line Generator             : GMG-30

Line Number                : 16250N

Line Array Type            : Dipole-Dipole

Line Data Type             : Complex Resistivity

Line A Spacing             : 100

Line Base Frequency        : .125

Line Job Number            : 307

Line Operator              : Justin Freytag

Line Tx Model              : GGT-30

Line Tx Serial             : 2001

Line Rx Model              : GDP-32

Line Rx Serial             : 3027

Line Rx EPROM              : ZMT-32 Z203b

Line Generator             : GMG-30

!----------------------------!

! Geographical Information   !

!----------------------------!

Survey Map Sheets          : Willuna

Survey Map Projection      : AMG

Map Zone number            : 51

Survey Hemisphere          : Southern

Survey Map Datum           : AGD66

Survey Spheroid            : ANS

Grid Azimuth               : 338.45

Local grid X origin        : 7730.89990234375

Local grid Y origin        : 13065.099609375

Map X origin               : 235811.3

Map Y origin               : 7018688.5

!----------------------------!

! User Defined Thresholds    !

!----------------------------!

Mask Description           : WR Default

Mask File Name             : wr.udt

Number of user overrides   : 7

!----------------------------!

! Type A Thresholds          !

!----------------------------!

Minimum Current                    :  2

Maximum Current                    :  30

Minimum Signal                     :  .001

Maximum Signal                     :  10000

Raw Phase Standard Deviation       : Null

Raw Phase SEM                      :  3

Maximum Contact Resistance         :  10

Minimum Cycles                     :  12

Minimum Bits of Resolution         :  9

Minimum Instrument Battery Voltage :  11

Maximum EM Coupling                :  30

!----------------------------!

! Type B Thresholds          !

!----------------------------!

Minimum Number of Repeats                                 :  2

Max Apparent Resistivity Standard Deviation               :  10

Max Diff Between Mean and Median for Apparent Resistivity : Null

Max Diff Between Max and Min for Apparent Resistivity     : Null

Max Apparent Resistivity Standard Error                   : Null

Max Raw Phase Standard Deviation                          :  2

Max Diff Between Mean and Median for Raw Phase            : Null

Max Difference Between Max and Min for Raw Phase          : Null

Max Raw Phase Standard Error                              : Null

Max Decoupled Phase Standard Deviation                    :  2

Max Diff Between Mean and Median for Decoupled Phase      : Null

Max Difference Between Max and Min for Decoupled Phase    : Null

Max Decoupled Phase Standard Error                        : Null

!----------------------------!

! Topographic Information    !

!----------------------------!

Survey Elevation           : 100

Topography Comment Line    : test topo data

Number of Data Lines       : 15

LINE 10400

8800 10400 100

8900 10400 100

9000 10400 100

9100 10400 100

9200 10400 100

9300 10400 100

9400 10400 100

9500 10400 100

9600 10400 100

9700 10400 100

9800 10400 100

9900 10400 100

10000 10400 100

10100 10400 100

!----------------------------!

! User Comments              !

!----------------------------!

Calibrate checks suggest channel 6 is noisy. Not used.

!----------------------------!

! Data Statistics            !

!----------------------------!

Min       Max      Mean    Median   Std Dev   Std Err

Transmitter Current        13.00     30.00     23.35     23.00      4.37     18.72

Instrument Voltage         11.50     12.40     12.09     12.20      0.30      2.45

Contact Resistance          0.00      6.58      1.41      0.99      1.29     92.04

Signal Strength             0.33     55.03      6.86      3.03      9.17    133.67

Apparent Resistivity        1.15      6.19      2.96      2.65      1.21     40.84

Decoupled Phase            -4.40     30.30      5.86      7.20      5.38     91.81

Raw Phase                   1.50     46.10     19.19     19.50      9.83     51.20

SEM                         0.00      6.93      0.41      0.12      0.80    195.42

EM Coupling                 1.70     31.20     13.33     12.60      6.53     49.01

!----------------------------!

! End of Survey Header       !

!----------------------------!

Line Number,Data Type,Block Number,Reading Channel,C1,C2,P1,P2,Local East Plot Point,Local North Plot Point,Map East Plot Point,Map North Plot Point,n Level,Base Frequency,Software Version,Number of Cycles,Transmitter Current,DC Phase,Status,Fundamental Magnitude,Raw Phase,Status,SEM,Standard Deviation,EM Coupling,Harmonic 3 Magnitude,Harmonic 3 Phase,Harmonic 5 Magnitude,Harmonic 5 Phase,Harmonic 7 Magnitude,Harmonic 7 Phase,Harmonic 9 Magnitude,Harmonic 9 Phase,Apparent Resistivity,Status,Gain,Total Gain,Attenuator,Bits of Resolution,Contact Resistance,Self Potential,Notch Filter,Instrument Voltage,Reading Date,Reading Time

,,,,m,m,m,m,,,,,m,m,m,m,m,m,m,m,,Hz,,,A,mrad,,mV,mrad,,mrad,mrad,,mV,mrad,mV,mrad,mV,mrad,mV,mrad,Ohm-m,,,,,,kOhm,mV,,V,,

16200N,RAW,53,4,9900,10000,9500,9600,9750.0,16200,236537.8,7022346.0,3,0.1250,0521,46,20.0,3.2000,PASS,3.7747,17.8000,PASS,0.04,0.27,14.60,3.7624,46.9000,3.7519,75.6000,3.7397,104.0000,3.7273,132.0000,2.7941,PASS,”063”,512,OUT,16,4.85,-36.9,OUT,11.50,94-08-06,11-18-17

16200N,RAW,54,4,9900,10000,9500,9600,9750.0,16200,236537.8,7022346.0,3,0.1250,0521,33,20.0,3.1000,PASS,3.7837,17.7000,PASS,0.05,0.29,14.60,3.7711,46.8000,3.7601,75.6000,3.7481,104.1000,3.7362,132.2000,2.8008,PASS,”063”,512,OUT,16,4.85,-36.9,OUT,11.50,94-08-06,11-24-35

,MIN,,,,,,,,,,,,,,,20.0,3.1000,,3.7747,17.7000,,,,17.7000,3.7624,46.8000,3.7519,75.6000,3.7397,104.0000,3.7273,132.0000,2.7941,,,,,,,,,,

,MAX,,,,,,,,,,,,,,,20.0,3.2000,,3.7837,17.8000,,,,17.8000,3.7711,46.9000,3.7601,75.6000,3.7481,104.1000,3.7362,132.2000,2.8008,,,,,,,,,,

,AVG,,,,,,,,,,,,,,,20.0,3.1500,,3.7792,17.7500,,,,17.7500,3.7667,46.8500,3.7560,75.6000,3.7439,104.0500,3.7318,132.1000,2.7974,,,,,,,,,,

,MED,,,,,,,,,,,,,,,20.0,3.1500,,3.7792,17.7500,,,,17.7500,3.7667,46.8500,3.7560,75.6000,3.7439,104.0500,3.7318,132.1000,2.7974,,,,,,,,,,

,SE,,,,,,,,,,,,,,,0.0,1.5873,,0.1191,0.2817,,,,0.2817,0.1155,0.1067,0.1092,0.0000,0.1122,0.0481,0.1192,0.0757,0.1191,,,,,,,,,,

,SDEV,,,,,,,,,,,,,,,0.0,0.0500,,0.0045,0.0500,,,,0.0500,0.0044,0.0500,0.0041,0.0000,0.0042,0.0500,0.0045,0.1000,0.0033,,,,,,,,,,

16200N,RAW,53,3,9900,10000,9600,9700,9800.0,16200,236584.3,7022365.0,2,0.1250,0521,46,20.0,0.3000,PASS,8.1414,11.9000,PASS,0.01,0.07,11.60,8.1339,35.0000,8.1243,57.7000,8.1117,80.1000,8.0981,102.4000,2.4106,PASS,”062”,256,OUT,16,6.13,-36.5,OUT,11.50,94-08-06,11-18-17

16200N,RAW,54,3,9900,10000,9600,9700,9800.0,16200,236584.3,7022365.0,2,0.1250,0521,33,20.0,0.3000,PASS,8.1612,11.9000,PASS,0.02,0.11,11.60,8.1533,34.9000,8.1433,57.6000,8.1307,80.1000,8.1173,102.5000,2.4164,PASS,”062”,256,OUT,16,6.13,-37.0,OUT,11.50,94-08-06,11-24-35

,MIN,,,,,,,,,,,,,,,20.0,0.3000,,8.1414,11.9000,,,,11.9000,8.1339,34.9000,8.1243,57.6000,8.1117,80.1000,8.0981,102.4000,2.4106,,,,,,,,,,

,MAX,,,,,,,,,,,,,,,20.0,0.3000,,8.1612,11.9000,,,,11.9000,8.1533,35.0000,8.1433,57.7000,8.1307,80.1000,8.1173,102.5000,2.4164,,,,,,,,,,

,AVG,,,,,,,,,,,,,,,20.0,0.3000,,8.1513,11.9000,,,,11.9000,8.1436,34.9500,8.1338,57.6500,8.1212,80.1000,8.1077,102.4500,2.4135,,,,,,,,,,

,MED,,,,,,,,,,,,,,,20.0,0.3000,,8.1513,11.9000,,,,11.9000,8.1436,34.9500,8.1338,57.6500,8.1212,80.1000,8.1077,102.4500,2.4135,,,,,,,,,,

,SE,,,,,,,,,,,,,,,0.0,0.0000,,0.1214,0.0000,,,,0.0000,0.1191,0.1431,0.1168,0.0867,0.1170,0.0000,0.1184,0.0488,0.1214,,,,,,,,,,

,SDEV,,,,,,,,,,,,,,,0.0,0.0000,,0.0099,0.0000,,,,0.0000,0.0097,0.0500,0.0095,0.0500,0.0095,0.0000,0.0096,0.0500,0.0029,,,,,,,,,,

16200N,RAW,51,5,10000,10100,9500,9600,9800.0,16200,236584.3,7022365.0,4,0.1250,0521,53,23.0,4.6000,PASS,3.4547,23.5000,PASS,0.04,0.29,18.90,3.4365,60.4000,3.4211,96.3000,3.4051,131.8000,3.3884,166.8000,4.4474,PASS,”063”,512,OUT,16,4.87,-35.1,OUT,11.50,94-08-06,11-01-59

16200N,RAW,52,5,10000,10100,9500,9600,9800.0,16200,236584.3,7022365.0,4,0.1250,0521,32,23.0,4.8000,PASS,3.4495,23.5000,PASS,0.04,0.23,18.70,3.4313,60.4000,3.4157,96.4000,3.4000,131.7000,3.3823,166.7000,4.4407,PASS,”063”,512,OUT,16,4.87,-35.1,OUT,11.50,94-08-06,11-08-06

,MIN,,,,,,,,,,,,,,,23.0,4.6000,,3.4495,23.5000,,,,23.5000,3.4313,60.4000,3.4157,96.3000,3.4000,131.7000,3.3823,166.7000,4.4407,,,,,,,,,,

,MAX,,,,,,,,,,,,,,,23.0,4.8000,,3.4547,23.5000,,,,23.5000,3.4365,60.4000,3.4211,96.4000,3.4051,131.8000,3.3884,166.8000,4.4474,,,,,,,,,,

,AVG,,,,,,,,,,,,,,,23.0,4.7000,,3.4521,23.5000,,,,23.5000,3.4339,60.4000,3.4184,96.3500,3.4026,131.7500,3.3853,166.7500,4.4440,,,,,,,,,,

,MED,,,,,,,,,,,,,,,23.0,4.7000,,3.4521,23.5000,,,,23.5000,3.4339,60.4000,3.4184,96.3500,3.4026,131.7500,3.3853,166.7500,4.4440,,,,,,,,,,

,SE,,,,,,,,,,,,,,,0.0,2.1277,,0.0753,0.0000,,,,0.0000,0.0757,0.0000,0.0790,0.0519,0.0749,0.0380,0.0901,0.0300,0.0753,,,,,,,,,,

,SDEV,,,,,,,,,,,,,,,0.0,0.1000,,0.0026,0.0000,,,,0.0000,0.0026,0.0000,0.0027,0.0500,0.0026,0.0500,0.0031,0.0500,0.0033,,,,,,,,,,

# Database Preferences

IPBase allows you to specify a number of settings which control the data IPBase displays and how it displays it. These preferences are accessed from the File > Preferences menu item and are automatically saved by IPBase in the ipbase.prf file in the \windows\system directory.

## A. Output Variable

This menu item specifies the variable which will be displayed in the pseudosection display when you click on the Display Results button.

## B. Output Value

This menu item specifies the value for the default output variable to be displayed in the pseudosection display when you click on the Display Results button.

## C. Default Plot

This menu item specifies the plot which will be displayed from the repeats form when you click on the Display Plot button.

## D. Maximum N level

This menu item specifies the maximum n level that will be exported when generating output data files. It does not affect  the data is imported into the database or the data which is displayed.

## E. Plot Axis Limits

Clicking on this menu item brings up the Plot Limits form on which you can specify how the spectral and phase plots are to be displayed.

Figure 36 Plot axis limits form

The first check box on this form allows you to specify whether skipped records are displayed in the spectral and phase plots. The default is that the item is unchecked, so all readings will be displayed in the plots. If the check box is turned on, only those readings which are not skipped (either by the operator in the field or by the database user) will be displayed.

The next part of the form relates to the phase plot. If the Auto Scale Phase plot is selected, the minimum and maximum phase values for all repeats for the current plot point/n level are calculated and the phase plot is displayed using these limits. If the check box is not selected, the specified values are used as the axis limits. Selecting the auto scale option allows all phase values to be assessed quickly regardless of data range. Whereas turning auto scale off allows for rapid comparisons of the phase from station to station as they are all displayed at the same scale.

The final part of this form relates to the spectral plot. This part of the form has the same structure and function as the phase plot part except that there are now minimum and maximum values for both the imaginary and real axes.

## F. Decoupled Phase Computed Using

This menu item allows you to specify how you want the decoupled phase to be computed. The two methods available are Standard 3pt and Coggon decoupling and you have the option to apply the decoupling formula to either the magnitude and phase or to the real and imaginary components.

## G. Use Coloured Lines

Clicking on this menu item toggles the option on and off. If the option is turned off, all lines in the plots will be displayed using a standard black pen. If the option is selected, the lines will be displayed using a cyclical list of pen colours.

## H. Use Dashed Lines

Clicking on this menu item toggles the option on and off. If the option is turned off, all lines in the plots will be displayed using a solid line. If the option is selected, the lines will be displayed using a cyclical list of dashed line styles.

## H. Opening Picture

If you click on this menu item you will be prompted to select a bmp file to display on the opening form when IPBase starts. To remove the opening picture, either select another file, or edit the ipbase.prf file in the \windows\system directory and delete the specified picture file name.

# Database Options

IPBase provides a number of facilities to modify and delete information contained in the database. These options are accessed by clicking on the Database menu item and selecting the required action.

## A. Edit Header

This menu item takes you to the survey design utility and displays the survey information contained in the database. From here you can either save the current header into a new file for use with a new database or view the current survey information. If you view the survey information, you can modify some of the information, but some information (such as the a spacing, Survey Mode, Receiver Model and Array Type) cannot be changed.

## B. Edit Line Header

This menu item takes you to the line header form and allows you to change any of the information pertaining to any of the lines stored in the database. Simply select the required line from the drop-down list in the bottom left corner of the form and enter the required information. Once all changes have been made, click on the Store Changes button to save the new information in the database.

Figure 38 Line Header Edit Form

## C. Input Options

There are occasions when you need to modify the information contained in a dump file before it is stored into the database. An example of this might be when the operator enters incorrect information into the receiver, or if the station location numbers are actually truncated coordinates and you want to have the real coordinate stored in the database. A number of the common cases where modifications to the data contained in the dump file have been identified and implemented in IPBases. These options are accessed from the Database > Options menu item.

## 1. Select Channels to Import

Clicking on this menu item toggles it between on and off (default is off). If this menu item is checked when you click on the Store IP Data in Database button, you will be presented with a form which allows you to specify which channels of data will be imported into the database.

## 2. Scale Coordinates on Import

When you Click on this menu item you will be presented with a dialog box prompting you for the scale factor to apply to the station coordinate information contained in the dump file. When data is imported from the dump file, the station coordinates read from the dump file are multiplied by the specified scale factor before being stored in the database. For example if the station locations are UTM coordinates with the last digit removed, you would specify a scale factor of 10 to recover the correct UTM coordinate.

## 3. Override a Spacing for import

Clicking on this menu item toggles it between on and off (default is off). If this menu item is checked when you click on the Store IP Data in Database button, the a spacing value contained in the dump file will be ignored and the value specified in the survey header will be used instead.

Normally any data in the dump file collected using an a spacing different from that specified in the survey header would be ignored and not stored in the database. When the override a spacing menu item is selected, all data in the dump file is treated as though it was collected using the survey a spacing and apparent resistivities will be recalculated using the survey a spacing.

## D. Database Options

## There are times where you have applied various threshold and overrides to data in the database when you might want to clear all the changes made and start anew. The following options allow you to do this in various stages without forcing you to create a new database and reload the data files,

## 1.  Clear User Overrides in Database

When you click on this menu item, IPBase clears all user overrides for all readings for all lines in the database. IPBase does not ask for confirmation before proceeding, so if you click on this menu item by accident, you can restore the overrides by using the File > Quit menu item which discards all changes made in the current session and exits IPBase. Alternatively you can recover the database as it was when IPBase opened it for the current session by exiting IPBase and deleting or (better) moving the current mdb file to another location and renaming the bak file created by IPBase and giving it the extension mdb.

When you clear the overrides, IPBase recalculates the data statistics for the database but does not reapply the thresholds, so you will need to reapply the data thresholds before you can view or export the data.

## 2. Override Apparent Resistivity Thresholds

There have been occasions when data contained poor phase readings but where the apparent resistivity values were quite good. In such cases, the data quality thresholds were useful in masking out the bad phase values, but were too harsh on the apparent resistivity values. One way to overcome this would be to go through all the readings and turn the resistivity values back on. As this would be very tedious, IPBase instead provides you with the option to ignore the threshold information when exporting data from the database.

## 3. Remove Duplicate Records

In some cases when importing data from multiple dump files, IPBase can end up with duplicates of some readings. Most duplicates are picked up and ignored when the data is imported from the dump files, but if you find that you have duplicate readings in the database, clicking on this menu item will force IPBase to apply a more stringent duplicate identification process and remove any duplicate record found.

## E. Recalculate Apparent Resistivity

When IPBase imports data from the dump file it automatically recalculates the apparent resistivity value before it is stored in the database. This is done as the information used in calculating the apparent resistivity in the instrument may have been entered incorrectly (such as station location or a spacing). The data file may have been edited to fix the error, but the apparent resistivity will be incorrect. To avoid this problem IPBase recalculates the value. As a further guarantee that the apparent resistivity values stored in the database are correct, clicking on this button will go through the entire database and recalculate the apparent resistivity values.

## F. Recalculate Decoupled Phase

IPBase stores the decoupled phase value read from the dump file directly into the database. However, unlike apparent resistivity, there are a number of options for calculating decoupled phase, so IPBase provides you with the capability to recalculate the decoupled phase using a number of different methods. The methods available in IPBase are the standard 3 point decoupling and Coggon’s 3 point formula. IPBase also provides you with the option to apply the decoupling formula to the real and imaginary components or to the magnitude and phase components. Most of the time the standard 3 point formula applied to magnitude and phase is quite adequate (and this is the default), but there may be times when you want to use a different method. To recalculate the decoupled phase, first select the method and components from the File > Preferences > Decoupled Phase Computed Using menu item and then click on the Database > Recalculate Decoupled Phase menu item to perform the calculation. Once the calculation has been performed, the database statistics are recalculated, but the thresholds are not recalculated.

## G. Remove Phase Flips

In some extreme cases where phases approach –3141 or +3141, the method used to calculate the phases and decoupled phases in the IP receiver fail and you will find that phases suddenly flip by 6282 mrad as they cross the negative real axis. Such phase flips result in incorrect values for the decoupled phase or for the average of the decoupled phase values. If you have data which exhibits this behaviour, click on the Database > Remove Phase Flips menu item and IPBase will analyse all of the data in the database and remove the phase flips for both individual readings and for all repeats at a particular station and store the modified values in the database. Note that IPBase automatically checks for phase flips when calculating averages for display and export, so this menu item is mainly of use to make the phase and spectral plots more meaningful.

## H. Import IP Data

This menu item allows you to select extra dump files to import into the database. Clicking on this menu item takes you to the Import Data screen described above.

## I. Import Archive File

This menu item is provided to allow you to import an archive file into the current database. When you click on this menu item you will be prompted to select the appropriate archive file to be imported. Once a valid archive file has been selected, the data contained in it will be imported into the database.

## J. Delete Survey Line

There are occasions when the wrong survey data is imported into a database. If this cannot be fixed by quitting the current session, the incorrect survey lines can be removed by clicking on the survey line in the list displayed when this menu item is selected. Note that IPBase will not delete the last remaining line In a database ie you cannot create an empty database by removing all the survey lines.

Appendix I

List of Tutorial files

File	Description

16200N.RAW	ZERO GDP-16 CR data collected by Tesla-10 (1994)

0.125Hz, 100m dipole-dipole array

16250N.RAW	ZERO GDP-32 CR data collected by ZERO (1996)

0.125Hz, 100m dipole-dipole array

HMOON.MDB	Honeymoon Well database.

HMOON.IPH	Honeymoon Well header file.

HMOON.ROT	Honeymoon Well grid location file.

WR.UDT	User Defined Thresholds defined by Western region geophysicists.