IPBASE CR GRADIENT ARRAY TUTORIAL

# Introduction

This document describes the process of running IPBase to read, assess, mask and output frequency domain gradient array data. It assumes that IPBase is installed on yous computer and that the sample data listed in Appendix I is available. Most of the basic information on running IPBase is already described in the CR Dipole-dipole tutorial document and so only those details specific to gradient array data will be presented here.

# Defining the Survey Header

IPBASE requires a header file to accompany any IP data.  You are given the option to create a new header or to read or modify an existing header file.  For the demonstration, you should select the Read Existing Header File option and select the file wabu.iph. Once the file is read you will be presented with the General survey information form displayed in Figure 1.

## General Survey Information

Figure 1 Survey Design Utility - General Survey Information screen

The main pieces of information which designate this as a frequency domain gradient array survey are the Survey Mode and Array Type items. The Instrument Type and a Spacing fields are also mandatory fields and show that the data was collected using a GDP-16 receiver using an a spacing of 50m. Once you are comfortable with the information displayed, click on the Done button to return to the main survey header form and click on Done again to return to IPBase.  You will then be prompted for a database name and the location to store the Access database file.

# IMPORTING THE DATA

After the Access database file has been created, you are then moved to the Import IP Data menu item (Figure 2).  The Receiver Type, Array Type, Data Type, a Spacing and Base Frequency to Read fields indicate the type of input data IPBASE is expecting, based on information provided within the Header.  For the tutorial, IPBASE is expecting GDP-16 CR data collected using 50m gradient array.  If any of this information other than the Receiver Type is not correct, you should quit IPBase without saving and start again.

Figure 2 IP data input screen following data input of 501000.RAW

Next you should select the data file to import.  For the tutorial select dump file 501000.RAW.  This data comes from Wabu for Freeport.  After the file is selected IPBase will scan the file and will report any problems found. If  any problems are found, you will need to edit the file to fix the problem before the data will be stored in the database. While scanning the file IPBase determines generates a list of the frequencies used in gathering the data. If there is more than one frequency, you will need to select which frequency to store in the database before the data will be saved. The file only contains data collected at 0.125 Hz so IPBase automatically stores the data.

Once the data is imported, IPBASE displays various details regarding the blocks read.  For example, in Figure 2 we see that there were 76 readings with 1 block skipped by the operator.

For the purposes of the tutorial, you should now import the files 501250.RAW and 501500.RAW.  This data is also 50m gradient array CR data .

# DATA STATISTICS

After all three files have been imported, you can select the Statistics menu item where a summary table of all the data currently contained within the database is presented (Figure 3).

Figure 3 Statistical summary screen

Points to note about the Statistics screen:

- The summary information may be misleading if the database contains more that one line.  You may view the statistics of one or more lines by using the Select Lines button.

- When the statistics the three files are viewed, the Rx EPROM field indicates that various EPROMS are in the database.  The correct EPROM value is indicated when only one data set is selected.  Note: No EPROM information available with GDP-16 software older than version 0527.

- Other functions of IPBASE, (i.e. output) are not influenced if you selects a subset of the lines to view.

- IPBASE will only handle data collected with one base frequency.

- To be consistent with other packages, phase information has been multiplied by -1

One interesting feature of this data is evident in the statistics for the raw and decoupled phases. The extreme range indicates that there is a problem with some of the phase values, this problem and how to deal with it will be examined in more detail later.

# DATA THRESHOLDS

By selecting Display under the Thresholds menu item, you are presented with the current set of Type A You Defined Thresholds which are all set to null and so are not displayed here.

The current Type B thresholds are presented by selecting the Show Type B button and are displayed in Figure 4.

Figure 4 Type B UDT screen

Points to note about these Thresholds screens:

- The Current Thresholds and Description are provided to indicate the current mask applied on the database.

- It is envisaged that users will have a specific set of UDT for a given area.  These can be either be hand entered or read from a file via the Thresholds menu.  Thresholds can also be entered via the Survey Design Utility.

- Under the Thresholds menu item, you always have the opportunity to save their UDT in a new file.

- At any stage you may clear the thresholds.  In addition, IPBASE provides you with two sets of “built in” UDT.  The first of these is null (the same as Clear Thresholds) and the second is a default set.

- In the Type B UDT screen, you are provided with a statistical summary of valid apparent resistivity, raw phase and decoupled phase.  This information is provided to give you an idea of appropriate Type B thresholds to apply.

- Total Plot Points refers to the number of unique (line number, station location) locations on a plan view of the gradient array survey. Total data points refer to the sum of all readings taken at all stations.  Failed refers to the number of data points and plot points failing to pass the current Type A thresholds and Type B thresholds.  In addition, the number of data and plot points flagged by the contractor are also indicated.

- The failures are inclusive, i.e. readings flagged by the contractor are counted as failures for the Type A and B UDT.

We can immediately see that 4 readings were flagged by the operator in the field. In addition 15 readings at 3 plot points failed the raw phase repeatability threshold and 41 readings at 8 plot points failed the decoupled phase repeatability threshold.

# DISPLAYING THE GRADIENT ARRAY DATA

Within the Thresholds screen, you can select the Display Results button to view a plan view of the data within the database. When first run the default is for the average apparent resistivity to be displayed as in Figure 5. The default variable and value can be specified from the File > Preferences > Output Variable and File > Preferences > Output Value menu items.

Figure 5 Display results screen

To locate where the phase data is not repeating well, select Raw Phase and No. Valid and the plan view will show those locations at which the raw phase data failed the type B thresholds. This display shows three plot points where the raw phase has failed the repeatability thresholds.

Figure 6 Plan view valid readings for raw phase

Double clicking on the station 9561215 on line 501250 in the display presents you with the repeats data screen in Figure 7.

Figure 7 Repeats data screen

This screen shows that all data was passed by the operator in the field and the apparent resistivity values satisfy the standard deviation threshold. However the phase data shows significant variation and fails the standard deviation threshold. If you click with the right hand mouse button on the first status flag for the raw phase all the status flags will be switched from SKIPB to USEB and the standard deviation at the current location will be displayed in the bottom part of the grid. The standard deviation for the raw phase at this location is 5.18 which is significantly larger than the threshold value of 2. Similarly the decoupled phase standard deviation is 12.48 which is even worse, which confirms that there are real problems with the phase data at this location.

To find out if there is actually any useful phase data at this location click on the Display Plot Button and select the Spectral Plot to display the graph shown in Figure 8.

Figure 8 Spectral Plot

The spectral plot confirms that there is significant variation between the repeats at this location. On examining the graphs for the repeats it appears that the data for repeat 1 and repeat 5 are quite different from the remaining repeats. So click on the check boxes for these repeats on the right hand side of the screen to turn the data type flag for the readings from RAW to SKIP. Once these readings have been turned off the spectral plot will now look like that displayed in Figure 9.

Figure 9 Spectral Plot with two repeats turned off

If your plot still displays the repeats which have been turned off, click on the File > Preferences > Plot Axis Limits menu item and turn off the Include Skipped Records in Plot check box. Then redisplay the plot by clicking on the Spectral Plot in the Graph list and it should now look like Figure 9.

Returning to the repeats data screen by clicking on the Display Data button, reveals that the data still fails the repeatability thresholds. If the SKIPB flags for repeats 2, 3 and 4 are overridden (by double clicking each one or clicking right on any of the SKIPB status flags), the standard deviations for the raw and decoupled phases are now 3.36 and 4.57 respectively. These are still worse than the threshold value of 2, but are probably acceptable.

Figure 10 Spectral Plot for line 501000 station 9561225

The next station to consider is at station 9561225 on line 501000 the spectral plot for which is displayed in Figure 10. The spectral response at this station displays almost no repeatability and there appears to be no reason to override the SKIPB flags.

The final station to examine is station 9561275 on line 501000 which is displayed in Figure 11 with the data for repeats 1 and 2 switched off.

Figure 11 Spectral and Phase Plot for line 501000 station 9561275

In this case the remaining readings show good repeatability and could probably be included by overriding the SKIPB flags were it not for the problem evident in the  phase plot. The base level of 3250 in the phase data means that the data cannot be used as it stands. The simplest and safest way to deal with such data is to ignore it. However IPBase does provide an option for repairing such phase reversed data under the Database > Remove Phase Reversals menu item. The use of this option should only be considered as a last resort and is provided for use on that basis. When phase reversals are removed the data is modified in the database and can only be restored to the original values by deleting the lines which have been modified and reloading the original raw data.

It is left as an exercise for you to locate the readings flagged by the operator in the file and to assess the quality of the flagged readings.

# Outputting data

Once the gradient array data has been assessed and bad data masked out using the data thresholds and you overrides applied, the next task is to output the data into one of the standard formats used for displaying and/or modelling. The data output options appropriate for gradient array data are:

- GEOSOFT DAT

- XYZ data file,

- Zonge AVG

- IPBASE archive (*.IPA).

## A. GEOSOFT

The file produced using the DAT (repeats reduced) format for the the apparent resistivity and decoupled phase data from the Wabu database  is:

IP DATA FROM : Wabu.mdb (IPBASE  V2.09) 8/10/1998 MASK:WABU.UDT

LINE: All ARRAY:GRAD DIPOLE:50 UNITS:M

F1        F2        M1        M2       RES       0IP

9562500.0 9560500.0 9561000.0 9561050.0    268.54     16.55

9562500.0 9560500.0 9561000.0 9561050.0    398.80     19.72

9562500.0 9560500.0 9561000.0 9561050.0    523.13     47.42

9562500.0 9560500.0 9561050.0 9561100.0    123.86     17.95

9562500.0 9560500.0 9561050.0 9561100.0    201.20     27.11

9562500.0 9560500.0 9561050.0 9561100.0    229.81         *

9562500.0 9560500.0 9561100.0 9561150.0    152.03     19.91

9562500.0 9560500.0 9561100.0 9561150.0    285.44         *

9562500.0 9560500.0 9561100.0 9561150.0    244.60         *

9562500.0 9560500.0 9561150.0 9561200.0    184.44     19.77

9562500.0 9560500.0 9561150.0 9561200.0    184.04     24.23

9562500.0 9560500.0 9561150.0 9561200.0    137.61     51.87

9562500.0 9560500.0 9561200.0 9561250.0    267.66     25.87

9562500.0 9560500.0 9561200.0 9561250.0    123.55     39.63

9562500.0 9560500.0 9561200.0 9561250.0      5.76         *

9562500.0 9560500.0 9561250.0 9561300.0    486.91     26.16

9562500.0 9560500.0 9561250.0 9561300.0    200.67         *

9562500.0 9560500.0 9561250.0 9561300.0     47.98         *

9562500.0 9560500.0 9561300.0 9561350.0    466.82     24.81

9562500.0 9560500.0 9561300.0 9561350.0    356.18     36.41

9562500.0 9560500.0 9561300.0 9561350.0    448.63         *

9562500.0 9560500.0 9561350.0 9561400.0    480.00     27.91

9562500.0 9560500.0 9561350.0 9561400.0    591.31     33.75

## B. XYZ

As gradient array data is usually collected on a grid of locations on the surface the xyz output option is appropriate for outputting the data. In this format the selected data is output along with the line and station location information into a Geosoft compatible xyz data file. Clicking on the Output Data > XYZ menu item brings up the form displayed in Figure 12.

Figure 12 XYZ output screen

The list of data variables and output values is identical in structure to the list for Geosoft output, and is used in exactly the same way. The main difference from the Geosoft output form is the Lines are East-West / Lines are North-South option items. Generally IPBase has no means of determining the direction of the data lines data stored in the database. Thus when the data is output you can explicitly specify the direction of the lines so that the line and station locations are correctly assigned to the x and y coordinate in the data file. For the lines in the Wabu survey the xyz output generated is :

LINE   501500.0

/                       Avg RES    Avg 0IP

9561025.0  501500.0     268.54      16.55

9561075.0  501500.0     123.86      17.95

9561125.0  501500.0     152.03      19.91

9561175.0  501500.0     184.44      19.77

9561225.0  501500.0     267.66      25.87

9561275.0  501500.0     486.91      26.16

9561325.0  501500.0     466.82      24.81

9561375.0  501500.0     480.00      27.91

9561425.0  501500.0     492.72      29.72

9561475.0  501500.0     412.77      25.43

9561525.0  501500.0     376.32      28.80

9561575.0  501500.0     319.50      28.05

9561625.0  501500.0     289.91      28.75

9561675.0  501500.0     237.02      28.71

9561725.0  501500.0     161.72      31.32

9561775.0  501500.0     199.39      32.95

9561825.0  501500.0     169.36      35.43

9561875.0  501500.0     150.27      33.70

9561925.0  501500.0     163.28      33.88

9561975.0  501500.0     202.33      34.01

LINE   501250.0

9561025.0  501250.0     398.80      19.72

9561075.0  501250.0     201.20      27.11

9561125.0  501250.0     285.44          *

9561175.0  501250.0     184.04      24.23

9561225.0  501250.0     123.55      39.63

9561275.0  501250.0     200.67          *

9561325.0  501250.0     356.18      36.41

9561375.0  501250.0     591.31      33.75

9561425.0  501250.0     369.69      31.35

9561475.0  501250.0     317.18      26.94

9561525.0  501250.0     464.63      26.48

9561575.0  501250.0     267.51      28.26

9561625.0  501250.0     112.36      26.00

9561675.0  501250.0     139.48      27.85

9561725.0  501250.0     300.85      31.52

9561775.0  501250.0     315.82      33.78

9561825.0  501250.0     241.74      33.29

9561875.0  501250.0     204.56      32.45

9561925.0  501250.0     157.79      31.95

9561975.0  501250.0     200.05      33.55

LINE   501000.0

9561025.0  501000.0     523.13      47.42

9561075.0  501000.0     229.81          *

9561125.0  501000.0     244.60          *

9561175.0  501000.0     137.61      51.87

9561225.0  501000.0       5.76          *

9561275.0  501000.0      47.98          *

9561325.0  501000.0     448.63          *

9561375.0  501000.0     385.52      47.78

9561425.0  501000.0     357.39      49.31

9561475.0  501000.0     184.82      43.45

9561525.0  501000.0      35.85      43.49

9561575.0  501000.0      44.36      47.43

9561625.0  501000.0     138.98      37.51

9561675.0  501000.0     210.85      32.17

9561725.0  501000.0     160.21      32.30

9561775.0  501000.0     225.68      30.40

9561825.0  501000.0     256.94      28.24

9561875.0  501000.0     384.91      27.67

9561925.0  501000.0     211.04      31.40

9561975.0  501000.0     208.53      37.85

This data can then be gridded and plotted in Geosoft.

## C. ZONGE

The Zonge output format data for the Wabu database  is:

\ IPBASE  V2.09 : Wabu.mdb Created on : 8/10/1998 USING : Mean Survey Type : Gradient Array

$ ASPACE=  50.0m

skp     Tx       Rx    PltPt    NSp  Freq Cmp Amps Resistivity  Phase    Real       Imag      %Rho   sPhz

\

2 9561000.0 9561100. 9561075.   1.0 0.000 Ex  2.8   1.2386e+2    18.0  1.0000e+0  0.0000e+0    0.0    0.2

2 9561000.0 9561100. 9561075.   1.0 0.125 Ex  2.8   1.2379e+1    14.9  1.0000e+0  1.4901e-2    0.0    0.0

2 9561000.0 9561100. 9561075.   1.0 0.375 Ex  2.8   1.2223e+1     8.6  9.8744e-1  8.4593e-3    0.0    0.2

2 9561000.0 9561100. 9561075.   1.0 0.625 Ex  2.8   1.2151e+1     1.9  9.8166e-1  1.8979e-3    0.0    0.0

2 9561000.0 9561100. 9561075.   1.0 0.875 Ex  2.8   1.2113e+1    -5.0  9.7856e-1 -4.8928e-3    0.0    0.0

2 9561000.0 9561100. 9561075.   1.0 1.125 Ex  2.8   1.2078e+1   -11.6  9.7567e-1 -1.1286e-2    0.0    0.1

2 9561050.0 9561150. 9561125.   1.0 0.000 Ex  2.8   1.5203e+2    19.9  1.0000e+0  0.0000e+0    0.0    0.2

2 9561050.0 9561150. 9561125.   1.0 0.125 Ex  2.8   1.3341e+1    16.0  1.0000e+0  1.5968e-2    0.0    0.1

2 9561050.0 9561150. 9561125.   1.0 0.375 Ex  2.8   1.3161e+1     8.2  9.8655e-1  8.1228e-3    0.0    0.1

2 9561050.0 9561150. 9561125.   1.0 0.625 Ex  2.8   1.3081e+1     0.7  9.8059e-1  6.8641e-4    0.0    0.2

2 9561050.0 9561150. 9561125.   1.0 0.875 Ex  2.8   1.3035e+1    -7.1  9.7716e-1 -6.9706e-3    0.0    0.1

2 9561050.0 9561150. 9561125.   1.0 1.125 Ex  2.8   1.3001e+1   -14.8  9.7448e-1 -1.4391e-2    0.0    0.3

2 9561050.0 9561650. 9561375.  11.0 0.000 Ex  2.8   3.8552e+2    47.8  1.0000e+0  0.0000e+0    0.0    0.6

2 9561050.0 9561650. 9561375.  11.0 0.125 Ex  2.8   1.5948e+1    39.3  1.0000e+0  3.9340e-2    0.0    0.2

2 9561050.0 9561650. 9561375.  11.0 0.375 Ex  2.8   1.5445e+1    23.1  9.6895e-1  2.2367e-2    0.0    0.3

2 9561050.0 9561650. 9561375.  11.0 0.625 Ex  2.8   1.5233e+1     7.8  9.5585e-1  7.4176e-3    0.0    0.4

2 9561050.0 9561650. 9561375.  11.0 0.875 Ex  2.8   1.5115e+1    -9.2  9.4845e-1 -8.7260e-3    0.1    0.9

2 9561050.0 9561650. 9561375.  11.0 1.125 Ex  2.8   1.5047e+1   -22.7  9.4398e-1 -2.1470e-2    0.1    0.9

2 9561100.0 9561200. 9561175.   1.0 0.000 Ex  2.8   1.8444e+2    19.8  1.0000e+0  0.0000e+0    0.0    0.0

2 9561100.0 9561200. 9561175.   1.0 0.125 Ex  2.8   1.4481e+1    16.5  1.0000e+0  1.6501e-2    0.0    0.0

2 9561100.0 9561200. 9561175.   1.0 0.375 Ex  2.8   1.4281e+1     9.8  9.8623e-1  9.6982e-3    0.0    0.0

2 9561100.0 9561200. 9561175.   1.0 0.625 Ex  2.8   1.4188e+1     3.0  9.7985e-1  2.9396e-3    0.0    0.1

2 9561100.0 9561200. 9561175.   1.0 0.875 Ex  2.8   1.4131e+1    -3.9  9.7591e-1 -3.8386e-3    0.0    0.0

2 9561100.0 9561200. 9561175.   1.0 1.125 Ex  2.8   1.4091e+1   -11.3  9.7312e-1 -1.0964e-2    0.0    0.2

………….

## D. ARCHIVE

The archive output for the above database is :

!--------------------------!

! Start of Survey Header   !

!--------------------------!

Header file for Wabu.mdb (IPBASE  V2.09) 8/10/1998

!----------------------------!

! General Survey Information !

!----------------------------!

Data type                  : Complex Resistivity

Array type                 : Gradient Array

A Spacing                  : 50

Survey Name                : WABU/HOLOMANA 1

Survey Contact             : MIKE BURSON (FREEPORT)

Survey Report Number       :

Cost Code                  :

Survey Line Number         : 501000,501250,501500

Survey Start Date          : 30/05/1996

Survey Finish Date         : 07/08/1996

Survey Cost                : 0

Survey Production Cost     : 0

Survey Production Rate     : 0

!----------------------------!

! Contractor Information     !

!----------------------------!

Survey Contractor          : ZERO

Survey Operator Name       : MARK van REED & GERRY  McNEIL

Survey Job Number          : 295

Number of Production Days  : 0

Number of Production Hours : 0

Survey Generator Model     : ZMG-7.5

Survey Tx Model            : GGT-10

Survey Tx Serial           : CR0509

Survey Rx Model            : GDP-16

Survey Rx Serial           : 1031/1040

Rx EPROM                   : GDP-16 G192q , ZMT-16 Z194b

Base Frequency             : .125

!----------------------------!

! Survey Line Information    !

!----------------------------!

Line Number                : 501000

Line Array Type            : Gradient Array

Line Data Type             : Complex Resistivity

Line A Spacing             : 50

Line Base Frequency        : .125

Line Job Number            : 295

Line Operator              : MARK van REED & GERRY  McNEIL

Line Tx Model              : GGT-10

Line Tx Serial             : CR0509

Line Rx Model              : GDP-16

Line Rx Serial             : 1031/1040

Line Rx EPROM              : GDP-16 G192q

Line Generator             : ZMG-7.5

Line Number                : 501250

Line Array Type            : Gradient Array

Line Data Type             : Complex Resistivity

Line A Spacing             : 50

Line Base Frequency        : .125

Line Job Number            : 295

Line Operator              : MARK van REED & GERRY  McNEIL

Line Tx Model              : GGT-10

Line Tx Serial             : CR0509

Line Rx Model              : GDP-16

Line Rx Serial             : 1031/1040

Line Rx EPROM              : GDP-16 G192q

Line Generator             : ZMG-7.5

Line Number                : 501500

Line Array Type            : Gradient Array

Line Data Type             : Complex Resistivity

Line A Spacing             : 50

Line Base Frequency        : .125

Line Job Number            : 295

Line Operator              : MARK van REED & GERRY  McNEIL

Line Tx Model              : GGT-10

Line Tx Serial             : CR0509

Line Rx Model              : GDP-16

Line Rx Serial             : 1031/1040

Line Rx EPROM              : ZMT-16 Z194b

Line Generator             : ZMG-7.5

!----------------------------!

! Geographical Information   !

!----------------------------!

Survey Map Sheets          :

Survey Map Projection      : UTM

Map Zone number            : 0

Survey Hemisphere          : Southern

Survey Map Datum           : WGC84

Survey Spheroid            : ANS

Grid Azimuth               : 10

Local grid X origin        : 725587.3125

Local grid Y origin        : 9584350

Map X origin               : 0

Map Y origin               : 0

!----------------------------!

! You Defined Thresholds    !

!----------------------------!

Mask Description           : SD > 2 for type B 0NLY

Mask File Name             : C:\PROJECTS\WABU\IPBASE\WABU.UDT

Number of you overrides   : 9

!----------------------------!

! Type A Thresholds          !

!----------------------------!

Minimum Current                    : Null

Maximum Current                    : Null

Minimum Signal                     : Null

Maximum Signal                     : Null

Raw Phase Standard Deviation       : Null

Raw Phase SEM                      : Null

Maximum Contact Resistance         : Null

Minimum Cycles                     : Null

Minimum Bits of Resolution         : Null

Minimum Instrument Battery Voltage : Null

Maximum EM Coupling                : Null

!----------------------------!

! Type B Thresholds          !

!----------------------------!

Minimum Number of Repeats                                 :  2

Max Apparent Resistivity Standard Deviation               :  2

Max Diff Between Mean and Median for Apparent Resistivity : Null

Max Diff Between Max and Min for Apparent Resistivity     : Null

Max Apparent Resistivity Standard Error                   : Null

Max Raw Phase Standard Deviation                          :  2

Max Diff Between Mean and Median for Raw Phase            : Null

Max Difference Between Max and Min for Raw Phase          : Null

Max Raw Phase Standard Error                              : Null

Max Decoupled Phase Standard Deviation                    :  2

Max Diff Between Mean and Median for Decoupled Phase      : Null

Max Difference Between Max and Min for Decoupled Phase    : Null

Max Decoupled Phase Standard Error                        : Null

!----------------------------!

! Topographic Information    !

!----------------------------!

Survey Elevation           : 0

Topography Comment Line    :

Number of Data Lines       : 0

!----------------------------!

! You Comments              !

!----------------------------!

!----------------------------!

! Data Statistics            !

!----------------------------!

Min       Max      Mean    Median   Std Dev   Std Err

Transmitter Current         2.80      2.80      2.80      2.80      0.00      0.00

Instrument Voltage         11.70     12.30     12.11     12.10      0.16      1.30

Contact Resistance          1.44     18.30      9.66      9.64      4.63     47.97

Signal Strength             0.25     35.24     15.70     14.76      8.24     52.47

Apparent Resistivity        5.69    591.44    260.37    229.87    134.44     51.64

Decoupled Phase            13.99   3347.32    108.95     32.05    487.98    447.88

Raw Phase                   9.20   3291.00    102.24     23.90    488.01    477.32

SEM                         0.01     27.40      0.81      0.09      2.52    310.82

EM Coupling                 0.00     63.20      7.98      6.90      7.61     95.33

!----------------------------!

! End of Survey Header       !

!----------------------------!

Line Number,Data Type,Block Number,Reading Channel,C1,C2,P1,P2,Base Line, Transmitter Line,Local East Plot Point,Local North Plot Point,Map East Plot Point,Map North Plot Point,n Level,Base Frequency,Software Version,Number of Cycles,Transmitter Current,DC Phase,Status,Fundamental Magnitude,Raw Phase,Status,SEM,Standard Deviation,EM Coupling,Harmonic 3 Magnitude,Harmonic 3 Phase,Harmonic 5 Magnitude,Harmonic 5 Phase,Harmonic 7 Magnitude,Harmonic 7 Phase,Harmonic 9 Magnitude,Harmonic 9 Phase,Apparent Resistivity,Status,Gain,Total Gain,Attenuator,Bits of Resolution,Contact Resistance,Self Potential,Notch Filter,Instrument Voltage,Reading Date,Reading Time

,,,,m,m,m,m,m,m,,,,,m,m,m,m,m,m,m,m,,Hz,,,A,mrad,,mV,mrad,,mrad,mrad,,mV,mrad,mV,mrad,mV,mrad,mV,mrad,Ohm-m,,,,,,kOhm,mV,,V,,

501000,RAW,89,4,9562500,9560500,9561000,9561050,501000,501500,50100,9561025.0,-669275.5,94326.5,501000,0.1250,0527,32,2.8,47.5750,PASS,26.2370,38.6000,PASS,0.02,0.11,9.00,25.4470,22.0000,25.1260,7.2000,24.9480,-7.5000,24.8360,-21.8000,523.1234,PASS,"005",32,OUT,16,15.50,-32.7,OUT,12.30,96-05-31,09-45-29

501000,RAW,90,4,9562500,9560500,9561000,9561050,501000,501500,50100,9561025.0,-669275.5,94326.5,501000,0.1250,0527,32,2.8,47.3375,PASS,26.2380,38.6000,PASS,0.02,0.11,8.80,25.4490,22.1000,25.1280,6.9000,24.9470,-7.8000,24.8360,-21.7000,523.1433,PASS,"005",32,OUT,16,15.50,-32.7,OUT,12.20,96-05-31,09-50-40

501000,RAW,91,4,9562500,9560500,9561000,9561050,501000,501500,50100,9561025.0,-669275.5,94326.5,501000,0.1250,0527,32,2.8,47.3375,PASS,26.2390,38.6000,PASS,0.02,0.11,8.80,25.4480,22.1000,25.1270,6.9000,24.9440,-7.5000,24.8360,-21.8000,523.1632,PASS,"005",32,OUT,16,15.50,-32.7,OUT,12.10,96-05-31,09-55-25

501000,RAW,92,4,9562500,9560500,9561000,9561050,501000,501500,50100,9561025.0,-669275.5,94326.5,501000,0.1250,0527,32,2.8,47.5000,PASS,26.2370,38.6000,PASS,0.03,0.17,8.90,25.4470,22.0000,25.1240,7.0000,24.9440,-7.5000,24.8370,-21.6000,523.1234,PASS,"005",32,OUT,16,15.50,-32.7,OUT,12.20,96-05-31,10-00-14

501000,RAW,93,4,9562500,9560500,9561000,9561050,501000,501500,50100,9561025.0,-669275.5,94326.5,501000,0.1250,0527,32,2.8,47.3750,PASS,26.2360,38.6000,PASS,0.02,0.11,8.80,25.4480,22.1000,25.1230,7.0000,24.9440,-7.6000,24.8320,-22.0000,523.1034,PASS,"005",32,OUT,16,15.50,-32.7,OUT,12.20,96-05-31,10-05-17

,MIN,,,,,,,,,,,,,,,,,2.8,47.3375,,26.2360,38.6000,,,,38.6000,25.4470,22.0000,25.1230,6.9000,24.9440,-7.8000,24.8320,-22.0000,523.1034,,,,,,,,,,

,MAX,,,,,,,,,,,,,,,,,2.8,47.5750,,26.2390,38.6000,,,,38.6000,25.4490,22.1000,25.1280,7.2000,24.9480,-7.5000,24.8370,-21.6000,523.1632,,,,,,,,,,

,AVG,,,,,,,,,,,,,,,,,2.8,47.4250,,26.2374,38.6000,,,,38.6000,25.4478,22.0600,25.1256,7.0000,24.9454,-7.5800,24.8354,-21.7800,523.1313,,,,,,,,,,

,MED,,,,,,,,,,,,,,,,,2.8,47.3750,,26.2370,38.6000,,,,38.6000,25.4480,22.1000,25.1260,7.0000,24.9440,-7.5000,24.8360,-21.8000,523.1234,,,,,,,,,,

,SE,,,,,,,,,,,,,,,,,0.0,0.2021,,0.0039,0.0000,,,,0.0000,0.0029,0.2221,0.0074,1.5649,0.0070,1.5385,0.0070,0.6091,0.0039,,,,,,,,,,

,SDEV,,,,,,,,,,,,,,,,,0.0,0.0959,,0.0010,0.0000,,,,0.0000,0.0007,0.0490,0.0019,0.1095,0.0017,0.1166,0.0017,0.1327,0.0203,,,,,,,,,,

This archive file contains all the information and you overrides stored in the database and can be imported into IPBase if required. If the header block is skipped, it can also be imported as a csv file into Excel.

Appendix I

List of Tutorial files

File	Description

501000.RAW	GDP-16 CR gradient array data 0.125Hz, 50m dipole-dipole array

501250.RAW	GDP-16 CR gradient array data 0.125Hz, 50m dipole-dipole array

501500.RAW	GDP-16 CR gradient array data 0.125Hz, 50m dipole-dipole array

WABU.MDB	Wabu gradient array  database.

WABU.IPH	Honeymoon Well header file.