IPBASE TD POLE-DIPOLE TUTORIAL

# Introduction

This document describes the process of running IPBase to read, assess, mask and output time domain pole-dipole data. It assumes that IPBase is installed on your computer and that the sample data listed in Appendix I is available. Most of the basic information on running IPBase is already described in the CR Dipole-dipole tutorial document and so only those details specific to pole-dipole data will be presented here.

# Defining the Survey Header

IPBASE requires a header file to accompany any IP data. You are given the option to create a new header or to read or modify an existing header file.  For the demonstration, you should select the Read Existing Header File option and select the file ipr.iph. Once the file is read you will be presented with the General survey information form displayed in Figure 1.

## General Survey Information

Figure 1 Survey Design Utility - General Survey Information screen

The main items of information which designate this as a time domain pole-dipole survey are the Survey Mode and Array Type items. The Instrument Type and a Spacing fields are also mandatory fields and show that the data was collected using a IPR-12 receiver using an a spacing of 50m. Once you are comfortable with the information displayed, click on the Done button to return to the main survey header form and click on Done again to return to IPBase.  You will then be prompted for a database name and the location to store the Access database file.

# IMPORTING THE DATA

After the Access database file has been created, you are then placed in the Import IP Data menu item (Figure 2).  The Receiver Type, Array Type, Data Type, a Spacing and Base Frequency to Read fields indicate the type of input data IPBASE is expecting, based on information provided within the Header.  For the tutorial, IPBASE is expecting IPR-12 pole-dipole data collected using 50m dipoles.  If any of this information other than the Receiver Type is not correct, you should quit IPBase without saving and start again.

Figure 2 IP data input screen following data input of L6.DAT

Next you should select the data file to import. For the tutorial select dump file L6.DAT. As this file is a GEOSOFT data file, IPBase will scan the header information in the file and will report any problems found. If any problems are found, you will need to edit the file to fix the problem before the data will be stored in the database. Once the header has been decoded IPBase automatically starts storing the data in the database. When a LINE: line is encountered, IPBase will bring up the Line Information Dialog box so that you can provide information pertinent to the data. The main difference for the pole-dipole array type is the information relating to the fixed transmitter electrode location. Where this information is given in the data file it is decoded and entered on the form. If the information is not given in the data file it is left blank. You should fill this information in if it is known, but IPBase will proceed even if the fields are left blank.

Figure 3 Line Identification Form

The next stage when storing Geosoft data in the database is the definition of the time windows used in collecting the IP data. Again this information is decoded from the file if it is available and entered on the time-window form shown in Figure 4.

Figure 4 Time Window Information

You now has the opportunity to enter or modify the time window start end and width, and also to define a start and end time for a user defined chargeability calculation (Mu Start and Mu End respectively). The total on-time and first channel delay time may also be changed if required.

The final information required by IPBase before the Geosoft data can be stored in the database is the association of the column names in the data file with the database field in which it is to saved. The Geosoft Data definition Form is displayed in Figure 5.

Figure 5 Geosoft Data Definitions

IPBase attemts to translate the Geosoft names to the corresponding database fields, but as there is no standard naming convention, you should ensure that the chosen Database name is correct. To change a Database Name, click on the relevant name to bring up a drop-down list of database field names and select the required name from the list. If a field in the data file does not need to be stored in the database, select the Database Name for that field and select Ignore from the list of database names.

This form also allows you to scale data columns before they are stored into the database. To change the scale value click on the relevant scale value to bring up the scale text edit box and type in the value required. Editing of the scale value can be cancelled by pressing the esc key.

Once the data is imported, IPBASE displays various details regarding the blocks read.  For example, in Figure 2 we see that there were 124 readings but no other useful information is contained in the Geosoft file, so there is no information available on skipped blocks or readings.

# DATA STATISTICS

After the data file has been imported, you can select the Statistics menu item where a summary table of all the data currently contained within the database is presented (Figure 6).

Figure 6 Statistical summary screen

# DATA THRESHOLDS

By selecting Display under the Thresholds menu item, you are presented with the current set of Type A User Defined Thresholds. All thresholds are set to null and this screen is displayed in Figure 7.

Figure 7 Type A thresholds Screen

Note that as this is time domain data there are a different set of thresholds from that for frequency domain data. As the data came from an IPR-12 receiver (albeit via a Geosoft data file), IPBase displays thresholds relevant to the IPR-12 available.

The Type B thresholds displayed in Figure 8 also display differences from the frequency domain form as the raw and decoupled phase columns are replaced with the mask channel M12 and the total chargeability M.

Figure 8 Type B Data Quality Thresholds

For the type B thresholds  the fields Total chargeabiliy (M) and a user selectable time window (M12) replace the raw and decoupled phase values for frequency domain data. The particular time window used for determining data repeatability is set at 12 for the IPR-12, but can be changed to any other time window by using the menu item Thresholds > Select Time Window and clicking on the required window.

# DISPLAYING THE POLE-DIPOLE DATA

With pole-dipole data there are two main conventions for defining the plot-point for displaying the data:

Geosoft Convention : the plot-point is half-way between the moving transmitter electrode and the nearest receiver electrode eg if T2Xis 0, R1X is 50 and R2X is 100 the plot-point will be located at 25.

Zonge Convention : the plot-point is half-way between the moving transmitter electrode and the centre of the receiver dipole eg if T2Xis 0, R1X is 50 and R2X is 100 the plot-point will be located at 37.5.

IPBase can work in either convention and the plot-point convention can be switched at any time. To change the plot-point convention click on the Database > Options > Pole-dipole plot point Convention menu item and select either Geosoft or Zonge.

Within the Thresholds screen, you can select the Display Results button to view a pseudosection view of the data within the database. When first run, the default is for the average apparent resistivity to be displayed as in Figure 9.

Figure 9 Pseudosection display of Apparent Resistivity

The default variable and value can be specified from the File > Preferences > Output Variable and File > Preferences > Output Value menu items.

Figure 10 Time Decay Plot

The quickest method for checking the quality of time domain data is to view the time decay plot for each of the plot points. To display this plot double-click on the first plot point in the pseudosection to bring up the repeats data screen and the click on the Display Plot button and select the Time Decay Plot from the Graph list box. The time decay plot for station 37.5 at n=1 is displayed in Figure 10.

As the data was imported from a Geosoft data file in which all repeats have already been reduced, there is only one repeat at each location and so the only option avaliable to you are to either accept or reject a reading.

One useful option when displaying the time decay plot is the ability to auto scale the displayed time decays. Clicking on the File Preference > Plot Axis Limits menu item brings up the form displayed in Figure 11.

Figure 11 Plotting Defaults

From this form you can select to auto scale the decay plot or to set minimum and maximum values to use. In Figure 11, the plot axis limits have been set as 0 and 20 which is the range of data displayed in Figure 10. Setting a fixed scale makes it easier to assess the variations in the decay across the pseudosection, while using the auto scale option makes it easier to asses decays at individual locations.

# Outputting data

Once the data has been assessed and bad data masked out using the data thresholds and you overrides applied, the next task is to output the data into one of the standard formats used for displaying and/or modelling. The data output options appropriate for pole-dipole data are:

- GEOSOFT DAT

- Zonge AVG

- IPBASE archive (*.IPA).

## A. GEOSOFT

The file produced using the DAT (repeats reduced) format for the the apparent resistivity and decoupled phase data from the database  is:

IP DATA FROM : line6.mdb (IPBASE  V2.09) 8/10/1998 MASK:ipr.udt

LINE:6E ARRAY:PLDP DIPOLE:50 UNITS:M  F2X:6.0 F2Y:-9999.0T=0,60,60,60,60,60,360,360,360,720,720,720

F1        F2        M1        M2       RES       0IP

0.0         *      50.0     100.0      9.36      1.82

0.0         *     100.0     150.0      6.65      0.03

50.0         *     100.0     150.0      7.80      3.68

0.0         *     150.0     200.0      7.32      0.06

50.0         *     150.0     200.0      7.52      1.29

0.0         *     200.0     250.0      9.28      1.54

100.0         *     150.0     200.0      7.02      1.27

50.0         *     200.0     250.0      8.81      2.36

0.0         *     250.0     300.0      5.27      5.91

100.0         *     200.0     250.0      8.07      2.67

50.0         *     250.0     300.0      4.61      5.22

0.0         *     300.0     350.0      6.04      3.89

150.0         *     200.0     250.0      6.68      2.85

100.0         *     250.0     300.0      4.19      5.73

50.0         *     300.0     350.0      5.55      3.95

150.0         *     250.0     300.0      3.39      6.13

100.0         *     300.0     350.0      4.73      5.63

50.0         *     350.0     400.0      7.11      8.16

200.0         *     250.0     300.0      2.97      4.41

150.0         *     300.0     350.0      4.13      4.39

100.0         *     350.0     400.0      6.45      9.55

200.0         *     300.0     350.0      3.79      4.17

150.0         *     350.0     400.0      5.41      9.46

100.0         *     400.0     450.0      7.07     10.19

250.0         *     300.0     350.0      9.31      4.96

200.0         *     350.0     400.0      4.90      8.00

150.0         *     400.0     450.0      6.18     10.20

250.0         *     350.0     400.0      6.21      4.46

200.0         *     400.0     450.0      5.44     10.12

150.0         *     450.0     500.0      8.49     18.76

300.0         *     350.0     400.0      7.96      4.34

250.0         *     400.0     450.0      5.71      9.02

200.0         *     450.0     500.0      7.93     17.83

300.0         *     400.0     450.0      4.79      7.25

250.0         *     450.0     500.0      8.31     15.29

200.0         *     500.0     550.0     13.64     17.58

350.0         *     400.0     450.0      5.51      8.93

300.0         *     450.0     500.0      7.04     14.29

250.0         *     500.0     550.0     14.75     17.18

350.0         *     450.0     500.0      6.73     15.52

300.0         *     500.0     550.0     13.32     16.47

250.0         *     550.0     600.0     13.92     18.69

400.0         *     450.0     500.0      6.94     14.95

350.0         *     500.0     550.0     12.69     15.71

300.0         *     550.0     600.0     12.87     19.63

400.0         *     500.0     550.0     14.57     16.87

350.0         *     550.0     600.0     12.03     16.67

300.0         *     600.0     650.0     18.01     18.22

450.0         *     500.0     550.0     24.39     20.14

## B. ZONGE

The Zonge output format data for the database  is:

\ CRAVG : line6.mdb (IPBASE  V2.09) 8/10/1998 USING : Mean

$ ASPACE=  50.0m

skp     Tx       Rx    PltPt    NSp  Freq Cmp Amps Resistivity  Phase    Real       Imag      %Rho   sPhz

2      0.00    50.00    37.50   1.0 0.000 Ex  2.7   9.3573e+0     1.8  1.0000e+0  0.0000e+0    0.0    0.0

2      0.00   100.00    62.50   2.0 0.000 Ex  2.7   6.6469e+0     0.0  1.0000e+0  0.0000e+0    0.0    0.0

2      0.00   150.00    87.50   3.0 0.000 Ex  2.7   7.3164e+0     0.1  1.0000e+0  0.0000e+0    0.0    0.0

2      0.00   200.00   112.50   4.0 0.000 Ex  2.7   9.2805e+0     1.5  1.0000e+0  0.0000e+0    0.0    0.0

2      0.00   250.00   137.50   5.0 0.000 Ex  2.7   5.2674e+0     5.9  1.0000e+0  0.0000e+0    0.0    0.0

2      0.00   300.00   162.50   6.0 0.000 Ex  2.7   6.0353e+0     3.9  1.0000e+0  0.0000e+0    0.0    0.0

2     50.00   100.00    87.50   1.0 0.000 Ex  2.8   7.7957e+0     3.7  1.0000e+0  0.0000e+0    0.0    0.0

2     50.00   150.00   112.50   2.0 0.000 Ex  2.8   7.5193e+0     1.3  1.0000e+0  0.0000e+0    0.0    0.0

2     50.00   200.00   137.50   3.0 0.000 Ex  2.8   8.8147e+0     2.4  1.0000e+0  0.0000e+0    0.0    0.0

2     50.00   250.00   162.50   4.0 0.000 Ex  2.8   4.6107e+0     5.2  1.0000e+0  0.0000e+0    0.0    0.0

2     50.00   300.00   187.50   5.0 0.000 Ex  2.8   5.5452e+0     3.9  1.0000e+0  0.0000e+0    0.0    0.0

2     50.00   350.00   212.50   6.0 0.000 Ex  2.8   7.1059e+0     8.2  1.0000e+0  0.0000e+0    0.0    0.0

2    100.00   150.00   137.50   1.0 0.000 Ex  2.3   7.0153e+0     1.3  1.0000e+0  0.0000e+0    0.0    0.0

2    100.00   200.00   162.50   2.0 0.000 Ex  2.3   8.0684e+0     2.7  1.0000e+0  0.0000e+0    0.0    0.0

2    100.00   250.00   187.50   3.0 0.000 Ex  2.3   4.1912e+0     5.7  1.0000e+0  0.0000e+0    0.0    0.0

2    100.00   300.00   212.50   4.0 0.000 Ex  2.3   4.7260e+0     5.6  1.0000e+0  0.0000e+0    0.0    0.0

2    100.00   350.00   237.50   5.0 0.000 Ex  2.3   6.4457e+0     9.5  1.0000e+0  0.0000e+0    0.0    0.0

2    100.00   400.00   262.50   6.0 0.000 Ex  2.3   7.0678e+0    10.2  1.0000e+0  0.0000e+0    0.0    0.0

2    150.00   200.00   187.50   1.0 0.000 Ex  3.3   6.6827e+0     2.8  1.0000e+0  0.0000e+0    0.0    0.0

2    150.00   250.00   212.50   2.0 0.000 Ex  3.3   3.3879e+0     6.1  1.0000e+0  0.0000e+0    0.0    0.0

2    150.00   300.00   237.50   3.0 0.000 Ex  3.3   4.1255e+0     4.4  1.0000e+0  0.0000e+0    0.0    0.0

2    150.00   350.00   262.50   4.0 0.000 Ex  3.3   5.4073e+0     9.5  1.0000e+0  0.0000e+0    0.0    0.0

2    150.00   400.00   287.50   5.0 0.000 Ex  3.3   6.1810e+0    10.2  1.0000e+0  0.0000e+0    0.0    0.0

2    150.00   450.00   312.50   6.0 0.000 Ex  3.3   8.4879e+0    18.8  1.0000e+0  0.0000e+0    0.0    0.0

2    200.00   250.00   237.50   1.0 0.000 Ex  4.6   2.9750e+0     4.4  1.0000e+0  0.0000e+0    0.0    0.0

2    200.00   300.00   262.50   2.0 0.000 Ex  4.6   3.7920e+0     4.2  1.0000e+0  0.0000e+0    0.0    0.0

2    200.00   350.00   287.50   3.0 0.000 Ex  4.6   4.9025e+0     8.0  1.0000e+0  0.0000e+0    0.0    0.0

2    200.00   400.00   312.50   4.0 0.000 Ex  4.6   5.4418e+0    10.1  1.0000e+0  0.0000e+0    0.0    0.0

2    200.00   450.00   337.50   5.0 0.000 Ex  4.6   7.9291e+0    17.8  1.0000e+0  0.0000e+0    0.0    0.0

2    200.00   500.00   362.50   6.0 0.000 Ex  4.6   1.3636e+1    17.6  1.0000e+0  0.0000e+0    0.0    0.0

2    250.00   300.00   287.50   1.0 0.000 Ex  4.3   9.3093e+0     5.0  1.0000e+0  0.0000e+0    0.0    0.0

2    250.00   350.00   312.50   2.0 0.000 Ex  4.3   6.2072e+0     4.5  1.0000e+0  0.0000e+0    0.0    0.0

2    250.00   400.00   337.50   3.0 0.000 Ex  4.3   5.7075e+0     9.0  1.0000e+0  0.0000e+0    0.0    0.0

2    250.00   450.00   362.50   4.0 0.000 Ex  4.3   8.3055e+0    15.3  1.0000e+0  0.0000e+0    0.0    0.0

2    250.00   500.00   387.50   5.0 0.000 Ex  4.3   1.4746e+1    17.2  1.0000e+0  0.0000e+0    0.0    0.0

2    250.00   550.00   412.50   6.0 0.000 Ex  4.3   1.3922e+1    18.7  1.0000e+0  0.0000e+0    0.0    0.0

2    300.00   350.00   337.50   1.0 0.000 Ex  3.4   7.9630e+0     4.3  1.0000e+0  0.0000e+0    0.0    0.0

2    300.00   400.00   362.50   2.0 0.000 Ex  3.4   4.7861e+0     7.2  1.0000e+0  0.0000e+0    0.0    0.0

2    300.00   450.00   387.50   3.0 0.000 Ex  3.4   7.0375e+0    14.3  1.0000e+0  0.0000e+0    0.0    0.0

2    300.00   500.00   412.50   4.0 0.000 Ex  3.4   1.3319e+1    16.5  1.0000e+0  0.0000e+0    0.0    0.0

2    300.00   550.00   437.50   5.0 0.000 Ex  3.4   1.2868e+1    19.6  1.0000e+0  0.0000e+0    0.0    0.0

2    300.00   600.00   462.50   6.0 0.000 Ex  3.4   1.8007e+1    18.2  1.0000e+0  0.0000e+0    0.0    0.0

2    350.00   400.00   387.50   1.0 0.000 Ex  5.2   5.5062e+0     8.9  1.0000e+0  0.0000e+0    0.0    0.0

2    350.00   450.00   412.50   2.0 0.000 Ex  5.2   6.7315e+0    15.5  1.0000e+0  0.0000e+0    0.0    0.0

## C. ARCHIVE

The archive output for the above database is :

!--------------------------!

! Start of Survey Header   !

!--------------------------!

Header file for line6.mdb (IPBASE  V2.09) 8/10/1998

!----------------------------!

! General Survey Information !

!----------------------------!

Data type                  : Time Domain

Array type                 : Pole-Dipole

A Spacing                  : 50

Survey Name                :

Survey Contact             :

Survey Report Number       :

Cost Code                  :

Survey Line Number         : 6E

Survey Start Date          : 30/12/1899

Survey Finish Date         : 30/12/1899

Survey Cost                : 0

Survey Production Cost     : 0

Survey Production Rate     : 0

!----------------------------!

! Contractor Information     !

!----------------------------!

Survey Contractor          :

Survey Operator Name       :

Survey Job Number          :

Number of Production Days  : 0

Number of Production Hours : 0

Survey Generator Model     :

Survey Tx Model            :

Survey Tx Serial           :

Survey Rx Model            : IPR-12

Survey Rx Serial           :

Rx EPROM                   :

On-time                    : 2000

First Window Delay Time    : 0

M0  Window Width          : 60

M1  Window Width          : 60

M2  Window Width          : 60

M3  Window Width          : 60

M4  Window Width          : 60

M5  Window Width          : 360

M6  Window Width          : 360

M7  Window Width          : 360

M8  Window Width          : 720

M9  Window Width          : 720

M10 Window Width          : 720

Mx Window Start            : 0

Mx Window End              : 0

Mu You Window Start       : 0

Mu You Window End         : 0

!----------------------------!

! Survey Line Information    !

!----------------------------!

Line Number                : 6E

Line Array Type            : Pole-Dipole

Line Data Type             : Time Domain

Line A Spacing             : 50

Line On-time               : 2000

Line Job Number            :

Line Operator              :

Line Tx Model              :

Line Tx Serial             :

Line Rx Model              : IPR-12

Line Rx Serial             :

Line Rx EPROM              :

Line Generator             :

Line Fixed Tx X            : 6.0

Line Fixed Tx Y            : -9999.0

!----------------------------!

! Geographical Information   !

!----------------------------!

Survey Map Sheets          :

Survey Map Projection      : AMG

Map Zone number            : 0

Survey Hemisphere          : Southern

Survey Map Datum           : WGC84

Survey Spheroid            : ANS

Grid Azimuth               : 0

Local grid X origin        : 0

Local grid Y origin        : 0

Map X origin               : 0

Map Y origin               : 0

!----------------------------!

! You Defined Thresholds    !

!----------------------------!

Mask Description           :

Mask File Name             : ipr.udt

Number of you overrides   : 0

!----------------------------!

! Type A Thresholds          !

!----------------------------!

Minimum Current                    : Null

Maximum Current                    : Null

Minimum Vp                         : Null

Maximum Vp                         : Null

Maximum Standard Deviation of M    : Null

Not Used                           : Null

Maximum Contact Resistance         : Null

Minimum Cycles                     : Null

Minimum Number of Wi               : Null

Not Used                           : Null

Not Used                           : Null

!----------------------------!

! Type B Thresholds          !

!----------------------------!

Minimum Number of Repeats                                 : Null

Max Apparent Resisitivty Standard Deviation               : Null

Max Diff Between Mean and Median for Apparent Resistivity : Null

Max Diff Between Max and Min for Apparent Resistivity     : Null

Max Apparent Resistivity Standard Error                   : Null

Max M12 Standard Deviation                               : Null

Max Diff Between Mean and Median for M12                 : Null

Max Difference Between Max and Min for M12               : Null

Max M12 Standard Error                                   : Null

Max Total Chargeability Standard Deviation                : Null

Max Diff Between Mean and Median for Total Chargeability  : Null

Max Difference Between Max and Min for Total Chargeability: Null

Max Total Chargeability Standard Error                    : Null

!----------------------------!

! Topographic Information    !

!----------------------------!

Survey Elevation           : 0

Topography Comment Line    :

Number of Data Lines       : 0

!----------------------------!

! You Comments              !

!----------------------------!

!----------------------------!

! Data Statistics            !

!----------------------------!

Min       Max      Mean    Median   Std Dev   Std Err

Transmitter Current         2.30      5.20      3.61      3.55      0.82     22.64

Primary Voltage             1.23    169.40     15.67      6.08     28.07    179.14

Contact Resistance          #N/A      #N/A      #N/A      #N/A      #N/A      #N/A

Apparent Resistivity        2.97     29.70      8.97      7.16      5.01     55.83

Total Chargeability        -3.51     35.23     13.90     14.94      7.79     56.02

Mx                          #N/A      #N/A      #N/A      #N/A      #N/A      #N/A

Mu                          #N/A      #N/A      #N/A      #N/A      #N/A      #N/A

Standard Deviation          #N/A      #N/A      #N/A      #N/A      #N/A      #N/A

M1                          4.40    183.40     81.46     84.20     40.18     49.32

M2                          2.80    124.50     54.26     57.75     26.30     48.46

M3                          1.20     99.40     43.16     46.60     21.42     49.63

M4                          1.20     87.10     36.80     40.00     18.67     50.72

M5                         -5.60     66.20     26.03     28.05     14.09     54.12

M6                        -12.70     47.30     17.55     18.85     10.32     58.80

M7                        -13.30     37.10     13.53     14.50      8.36     61.82

M8                        -12.00     28.30     10.19     11.05      6.68     65.52

M9                         -9.70     21.40      7.61      8.30      5.28     69.38

M10                        -8.30     18.70      6.07      6.65      4.46     73.54

M11                         #N/A      #N/A      #N/A      #N/A      #N/A      #N/A

!----------------------------!

! End of Survey Header       !

!----------------------------!

Survey Line,Data Type,Line Number,Reading Channel,C1,C2,P1,P2,Local East Plot Point,Local North Plot Point,Map East Plot Point,Map North Plot Point,n Level,On-time,Software Version,Number of Cycles,Transmitter Current,Total Chargeability,Status,Vp,M12,Status,Standard Deviation,Wi,Mx,Mu,M0,M1,M2,M3,M4,M5,M6,M7,M8,M9,M10,Apparent Resistivity,Status,Contact Resistance,Self Potential,Instrument Voltage,Reading Date,Reading Time

,,,,m,m,m,m,m,m,m,m,,s,,,A,mV/V,,mV,mV/V,,mV/V,,mV/V,mV/V,mV/V,mV/V,mV/V,mV/V,mV/V,mV/V,mV/V,mV/V,mV/V,mV/V,mV/V,Ohm - m, ,kOhm, mV,V, ,

6E,RAW,1,0,0,50,50,100,6,37.5,6.0,37.5,1,2000.0000,,0,2.7,1.8170,PASS,40.2100,-99999.0000,PASS,-99999.00,0,-99999.0000,-99999.0000,7.3000,5.6000,4.7000,4.1000,3.1000,2.3000,1.9000,1.5000,1.2000,1.0000,-99999.0000,9.3573,PASS,-99999.00,-34.0,-99999.00,98-02-15,16-18-40

,MIN,,,,,,,,,,,,,,,,1.8170,,40.2100,#N/A,,,,#N/A,#N/A,7.3000,5.6000,4.7000,4.1000,3.1000,2.3000,1.9000,1.5000,1.2000,1.0000,#N/A,9.3573,,,,,

,MAX,,,,,,,,,,,,,,,,1.8170,,40.2100,#N/A,,,,#N/A,#N/A,7.3000,5.6000,4.7000,4.1000,3.1000,2.3000,1.9000,1.5000,1.2000,1.0000,#N/A,9.3573,,,,,

,AVG,,,,,,,,,,,,,,,,1.8170,,40.2100,#N/A,,,,#N/A,#N/A,7.3000,5.6000,4.7000,4.1000,3.1000,2.3000,1.9000,1.5000,1.2000,1.0000,#N/A,9.3573,,,,,

,MED,,,,,,,,,,,,,,,,1.8170,,40.2100,#N/A,,,,#N/A,#N/A,7.3000,5.6000,4.7000,4.1000,3.1000,2.3000,1.9000,1.5000,1.2000,1.0000,#N/A,9.3573,,,,,

,SE,,,,,,,,,,,,,,,,0.0000,,0.0000,#N/A,,,,#N/A,#N/A,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,#N/A,0.0000,,,,,

,SDEV,,,,,,,,,,,,,,,,0.0000,,0.0000,#N/A,,,,#N/A,#N/A,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,#N/A,0.0000,,,,,

6E,RAW,1,0,0,50,100,150,6,62.5,6.0,62.5,2,2000.0000,,0,2.7,0.0298,PASS,9.5210,-99999.0000,PASS,-99999.00,0,-99999.0000,-99999.0000,4.4000,2.8000,1.2000,1.2000,0.2000,-0.5000,0.0000,0.1000,-0.2000,-0.3000,-99999.0000,6.6469,PASS,-99999.00,39.0,-99999.00,98-02-15,16-18-40

,MIN,,,,,,,,,,,,,,,,0.0298,,9.5210,#N/A,,,,#N/A,#N/A,4.4000,2.8000,1.2000,1.2000,0.2000,-0.5000,0.0000,0.1000,-0.2000,-0.3000,#N/A,6.6469,,,,,

,MAX,,,,,,,,,,,,,,,,0.0298,,9.5210,#N/A,,,,#N/A,#N/A,4.4000,2.8000,1.2000,1.2000,0.2000,-0.5000,0.0000,0.1000,-0.2000,-0.3000,#N/A,6.6469,,,,,

,AVG,,,,,,,,,,,,,,,,0.0298,,9.5210,#N/A,,,,#N/A,#N/A,4.4000,2.8000,1.2000,1.2000,0.2000,-0.5000,0.0000,0.1000,-0.2000,-0.3000,#N/A,6.6469,,,,,

,MED,,,,,,,,,,,,,,,,0.0298,,9.5210,#N/A,,,,#N/A,#N/A,4.4000,2.8000,1.2000,1.2000,0.2000,-0.5000,0.0000,0.1000,-0.2000,-0.3000,#N/A,6.6469,,,,,

,SE,,,,,,,,,,,,,,,,0.0000,,0.0000,#N/A,,,,#N/A,#N/A,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,#N/A,0.0000,,,,,

,SDEV,,,,,,,,,,,,,,,,0.0000,,0.0000,#N/A,,,,#N/A,#N/A,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,#N/A,0.0000,,,,,

6E,RAW,1,0,50,100,100,150,6,87.5,6.0,87.5,1,2000.0000,,0,2.8,3.6787,PASS,34.1200,-99999.0000,PASS,-99999.00,0,-99999.0000,-99999.0000,11.2000,8.6000,7.5000,6.6000,5.2000,4.1000,3.6000,3.2000,3.0000,2.7000,-99999.0000,7.7957,PASS,-99999.00,571.0,-99999.00,98-02-15,16-18-41

,MIN,,,,,,,,,,,,,,,,3.6787,,34.1200,#N/A,,,,#N/A,#N/A,11.2000,8.6000,7.5000,6.6000,5.2000,4.1000,3.6000,3.2000,3.0000,2.7000,#N/A,7.7957,,,,,

,MAX,,,,,,,,,,,,,,,,3.6787,,34.1200,#N/A,,,,#N/A,#N/A,11.2000,8.6000,7.5000,6.6000,5.2000,4.1000,3.6000,3.2000,3.0000,2.7000,#N/A,7.7957,,,,,

,AVG,,,,,,,,,,,,,,,,3.6787,,34.1200,#N/A,,,,#N/A,#N/A,11.2000,8.6000,7.5000,6.6000,5.2000,4.1000,3.6000,3.2000,3.0000,2.7000,#N/A,7.7957,,,,,

,MED,,,,,,,,,,,,,,,,3.6787,,34.1200,#N/A,,,,#N/A,#N/A,11.2000,8.6000,7.5000,6.6000,5.2000,4.1000,3.6000,3.2000,3.0000,2.7000,#N/A,7.7957,,,,,

,SE,,,,,,,,,,,,,,,,0.0000,,0.0000,#N/A,,,,#N/A,#N/A,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,#N/A,0.0000,,,,,

,SDEV,,,,,,,,,,,,,,,,0.0000,,0.0000,#N/A,,,,#N/A,#N/A,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,0.0000,#N/A,0.0000,,,,,

This archive file contains all the information and you overrides stored in the database but IPBase cannot reimport the data into a database, but is suitable for importing as a csv file into Excel.

Appendix I

List of Tutorial files

File	Description

L6.DAT	IPR-12 50m pole-dipole array

LINE6.MDB	Pole-dipole  database.

IPR.IPH	Honeymoon Well header file.