StorkGradientIPreport.doc

TO:		Mark Saxon

CC:

FROM:		David McInnes

DATE:  	February 19, 2009

SUBJECT:	STORKULLEN Gradient Array IP

The first line of data (LINE550N_LINE500N.txt) was recorded on the 30th of June 08.

It was emailed to the author on the 1st of July 08.

Some problems with the data were encountered – Principally an incorrect setting in the receiver rendering the calculated resistivity data invalid. Additionally the array set up for the southern block of data collection had significant implications to the collection and processing of the data.

The chargeability data appears to be less effected.

All the Gradient data was collected with an incorrect system setting, resulting in all the data having an incorrectly calculated resistivity, this in correct instrumament setting and its implications were highlighted in an email to Peder on the 3rd of July 2008 and replied on the 8th of July 2008 Below).

This renders it Unacceptable.

No data files with the system array corrected and resistivities recalculated have ever been supplied to the client or its representative.

Therefore no valid data has been supplied.

I authorised the payment of the northern data block, as I was able to easily correct and recalculate the data. This is due to the survey orientation being adjusted, moving to an East-West transmitter. However I was unhappy with the results of my recalculation for the southern block of data and remain so, as described below.

Hence I recommended at the time and remains today - no payment be made for its collection until a competent set of data is supplied by the contractor to the client or its representative.

Below is the email I sent to Peder on the 3rd of July 08, after first viewing the data; with the reply from Peder on the 8th of July 08. Following that are some screenshots of the software used to analysis the data integrity. These demonstrate the in correct calculations and the implications of the orientation of the survey array (i.e. the North South transmitter).

My email 3/07/08

Hi Peder,

Had a look at the data.
Just need a couple of things confirmed

In the header of the output file it still states that the data is Dipole-Dipole
The operator should change this to Gradient array as I am sure this setting wil be affecting the instruments calculations of the resistivity field.

The big question however -
is it correct that the Tx dipole is being set out in a NS orentation and the lies are being read in an east west orienation.
This is the first time I have seen this.
Normally I see the TX dipole being set out in the same orientation as the Rx dipoles.
And quite large so that with 25m dipoles you wouldn't get within 200m of either pit.

Can you confirm the Tx orientation?

Cheers
David

Reply on the 8/7/08

Hi David,

sending data collected until Monday.

Cheers,

Peder

The data was received in, “machine dump” format (as requested).

There were errors in some blocks of data, not significant, but they should have been corrected before the files were emailed to the client or its representative.

However, as previously stated, the incorrect array setting in the header of the file is significant. The calculation of the resistivity is directly related to the type of array used. Therefore the incorrect setting of the instrument results in an incorrect calculation of resistivity being stored in the machines dump file (the file supplied to the client).

The selection of figures below; are screen snap shots from the software used to analysis the quality of the IP data. They demonstrate the effects of having the incorrect setting. Changing the setting and recalculating the data should in essence be not to difficult. However a simple recalculation became difficult due to the array set up; North-South transmitter dipole.

This required me to fund modifications to my software to be able to process data with this orientation (it was their first time with this survey orientation as well). However THE resultant images of the processed data indicate the re-calculations are not correct.

The first gradient data looks like Figure 1

Figure 1: Displays the difference in the plotting of the data due to the incorrect survey set up (Dipole-Dipole not Gradient). The implications to the calculation of the resistivity due to the incorrect electrode positions can be drawn.

Figure 2a: Displaying the chargeability decays – despite the difference in amplitude they are very coherent. Note the different resistivity calculations due to the incorrect array information.

Figure 2b: Displays how the recalculation of the data can be obtained by correcting the collection geometry.

The initial recalculation of the resistivity data was incorrect due to my software not accepting the transmitter orthogonal to the data collection traverses. Once the software fix was received, the reprocessing of the resistivity calculations was undertaken. However the resultant images indicate that there are still some problems in the calculations (Figure 3a). Hence I can not approve the data’s integrity or validity.

Figure 3a: Image of the recalculated resistivity data. The large high in on line 400N around 700e suggests that there are some problems in the resistivity calculation. The recorded signal does not look abnormal (figure 3b).

Figure 3b: the recorded raw signal (Vp) looks fine, not the increased signal strength adjacent to the elctrodes.

Figure 4 displays the newly calculated resistivity for all the traverses. Note that all the resistivity values calculated for line 400N appear and order of magnitude higher then those for the adjacent lines.

Figure 5: Calculated resistivity image displaying the removal of the large resistivity values.

Figure 6: Image displaying the merge of the resistivity data  for the Northern and southern blocks of gradient data.